Gdansk University of Technology

Faculty of Electronics, Telecommunications and Informatics

# Multimedia Systems Department

# ul. Narutowicza 11/12,  80-952 Gdansk; Poland   tel. (58) 347-13-01;  fax (58) 347-11-14;                           e-mail: ksm@sound.eti.pg.gda.pl           http://www.multimed.org/

INTERNAL REPORT

of the current research and development status

title: „MTS tracker tests”

report no 01/2007

work done within the PRESTOSPACE project

time period from 01-11-2007 to 09-11-2007

Project manager: prof. dr hab. inż. A. Czyżewski

Author:  mgr inż. P. Maziewski

Gdansk, 09-11-2007

- Contents

- Purpose of this work

- Connections between this and previous works

- Work description

- Conclusions and further work directions

- Appendixes

- Purpose of this work

Purpose of this work was to test the developed algorithm for the MTS pilot tone tracking which was recently ported to the DeWower VST plug-in.

- Connections between this and previous works

This report resulted from the research and development as well as the implementation done by Piotr Pastuszak. The report is also closely connected with the work done by Maciek Szczodrak who was responsible for porting the algorithm to the DoWower VST.

- Work description

The MTS (Multichannel Television Standard) tone is available in the NTSC (National Television System Committee) stereophonic audio recordings. This tone can be tracked in order to determine the wow and flutter characteristic, i.e. the PVC (Pitch Variation Curve). Currently we have two wow distorted audio examples which contain this tone. Those sound samples, provided by BBC and INA (see the appendix), were used in the testing procedure.

The MTS tracked, researched and developed by Piotr Pastuszak, was recently ported to the Dewoer plug-in. Additionally, it was integrated with the high frequency bias tracker. Therefore the newly created algorithm called ‘The High Frequency Pilot Tone Tracker’ was tested not only using the NTSC (MTS) sound samples but also using the sound samples with the high frequency bias (see the appendix).

Additionally, the updated DeWower manual was studied and corrected.

Following problems/errors/bugs were found:

- The manual must be further corrected according the comments made in the doc file.

- The proposed MTS-based restoration scenario must be expand including information on how to handle the first wow distortion in the ‘wow.wav’ file (located between 0:03.767 and 0:06.116). Some manual processing will probably be needed.

- Fonts used to described the parameters labels in the DeWower GUI are too big (see the next picture).

- Name of the algorithm (in the pop-up menu) must by changed from ‘High Frequency Bias Tracking’ to ‘High Frequency Pilot Tone Tracking’.

- When applying following procedure: Create PVC (no.1), Import PVC (no.2), move to a next audio file, move back to the previous audio file; the current PVC is no.1 whereas it should be no.2. This must be corrected.

- Some debugging is necessary. For example the plug-in crashes when applying the algorithm on the MTS examples twice with the parameters set as shown in the following pictures (concern both of the tested MTS sound samples). This do not happens when the bias sample is processed.

- Conclusions and further work directions

The encountered problems must be solved. Then the new version of the algorithm must be tested again.

- Appendix

Sound samples description:

- Wow.wav – contains the MTS pilot tone. This recording was provided by JRS. It originates from the previous PRESTO project and was probably made by the BBC archives. This stereophonic recording is made of fragments taken from five different recordings.

- wf_INA1_48kHz.wav – contains the MTS pilot tone. This recording was provided by INA. It contains a short musical fragment with a singing vocal and few instruments.

- NAGRA_4 – contains the high frequency bias. This recording was made in the MSD laboratory.