Drift, Wow and Flutter Measurement and Reduction in Shrunken Movie Soundtracks.

P. Maziewski, A. Kupryjanow, A. Czyzewski

Abstract

Paper presents a set of algorithms for determining and reducing drift, wow and flutter (hereafter wow) in shrunken movie tapes. The idea behind the algorithms is to use image processing for calculating a local tape shrinkage which is one of the reasons for wow. Shrinkage can be calculated via analyzing the height of: a movie frame, sprocket hole, pitch or another standardized movie tape element; and then it can be expressed as a wow characteristic. After wow determination both the soundtrack and movie frames can be corrected. As wow reduction can be applied for the images and digital sound, algorithms for translating soundtrack images to digital waveforms are also given.

Précis

Most of films suffer from tape shrinkage. It is caused by loss of water, solvent and plasticizer in movie tapes. Both nitro based and acetate based films are exposed to this process. Only a fresh tape taken from the production line meets its normative requirements whereas the archive films all have different shrinkages. Such tape deformations are one of the reasons for wow distortions in the corresponding soundtrack. Additionally, shrinkage affects not only audio but also pictures, i.e. frames proportions are disturbed. In most cases tapes are uniformly shrunken, however, in some situations irregular deformations are possible. Those lead to accidental wow distortions which are highly perceptible and unpleasant. Thus, having a procedure for determining and reducing such distortions is of high importance.

The proposed paper presents a set of algorithms for determining and reducing audible and visible effects of movie tapes shrinkage. The algorithms can be used separately, e.g. only for wow reduction, however, the most convenient way is to use them in the following combination, which allows for the simultaneous sound and image restoration:

Figure.1 Algorithms set up for reducing audible and visible effects of movie tapes shrinkage.

The proposed processing is based on a frame-by-frame scheme. First the movie tape is scanned manually or automatically. Automatic scanning can be applied for tapes in good condition, i.e. mostly modern ones, whereas fragile and damaged archive films must be digitized manually. Next some initial preprocessing, i.e. contrast adjustments, mathematical morphology or other image processing techniques, can be applied to facilitate further processing. Wow characteristic determination (called the Pitch Variation Curve – PVC) is performed using the image features analysis such as the height of: a movie frame, sprocket hole or pitch. Consecutive heights - normalized using their standardized values - constitute the PVC. Based on the determined wow characteristic, the soundtrack images as well as the movie frames images can be corrected to compensate the shrinkage. Two-dimensional nonuniform resampling is used to obtain the restored pictures. The wow reduction can be also applied for the corresponding digital audio. Then it only requires an one-dimensional nonuniform resampling, which leads to a shorter computational time. For digital audio processing, algorithms for image to digital sound translation are needed. Thus they are given in the paper. At the end, the corrected sound and pictures can be further processed using some external tools and finally both can be combined into the restored digital movie.

The paper presents the theoretical study behind the proposed algorithms as well as the experiments which verify their performance. Theory reveals the connection between the newly proposed approach and the known techniques for wow determination and reduction. Experiments are focused on proving the theoretically evaluated performance. They involve genuine and artificially simulated tape shrinkages for the 16mm and 35mm films. Simulations are necessary to examine the maximal wow modulation frequency which can be captured in the determined PVC. All of the presented experiments prove the idea behind the proposed approach.

Paper outline will be as follows:

1. Introduction

2. Concept of Simultaneous Sound and Image Restoration

2.1 Image Preprocessing

2.2 Pitch Variation Curve Calculation

2.3 Image to Sound Translation

2.3.1 A-Chain

2.4 Movie Frames Correction

2.5 Combining Sound with Image and Additional Processing

3. Processing Examples

3.1 16mm film

3.2 35mm film

4. Summary

5. References