Gdansk University of Technology

Faculty of Electronics, Telecommunications and Informatics

# Multimedia Systems Department

# ul. Narutowicza 11/12,  80-952 Gdansk; Poland   tel. (58) 347-13-01;  fax (58) 347-11-14;                           e-mail: ksm@sound.eti.pg.gda.pl           http://www.multimed.org/

INTERNAL REPORT

of the current research and development status

title: „DeWower plug-in tests”

report no 04/2007

work done within the PRESTOSPACE project

time period from 26-11-2007 to 30-11-2007

Project manager: prof. dr hab. inż. A. Czyżewski

Author:  mgr inż. P. Maziewski

Gdansk, 16-11-2007

- Contents

- Purpose of this work

- Connections between this and previous works

- Work description

- Conclusions and further work directions

- References

- Purpose of this work

Purpose of this work was to test the DeWower plug-in which was upgraded according to the evaluators advises.

- Connections between this and previous works

This report refer to the previous reports on the DeWower plug-in tests and upgrades [1, 2].

- Work description

The upgraded plug-in was tested and following problems/errors/bugs were found (red colour – unsolved problems, blue – solved problems regarding the previous report [1]):

- The ‘overlap’ parameter in the high frequency pilot tone tracker cannot be set to 0 (as it is advised in the restoration scenario no.1 – see the DeWower  instruction).

- The ‘bottom margin’ parameter in the high frequency pilot tone tracker is not included in the restoration scenarios from 1 till 3 in the DeWower  instruction.

- After closing all the sound files the Dower is still active with the spectrogram of the last processed file. The user can now apply wow determination algorithms and manual routines (like zooming). This should be deactivated.

- The expression ‘bias tracker’ in restoration scenarios must be replaced by the ‘high frequency pilot tones tracker’

- Parameters set up described in the ‘Step 1’ in the restoration scenario no.3 do not generate same results as the one presented in the corresponding picture.

- The ‘overlap parameter cannot be set to 63.5% (only integer values are possible) as it is advised in restoration scenario no.3

- Parameters set up described in the ‘Step 4’ in the restoration scenario no.3 do not generate same results as the one presented in the corresponding picture

- Parameters set up described in the ‘Step 1’ in the restoration scenario no.4 do not generate same results as the one presented in the corresponding picture.

- In ‘Step 4’ in the restoration scenario no.4 the proposed COG algorithm do not allow to obtain a proper PVC

- ‘Step 3’ in the restoration scenario no.5 is useless (the resulting PVC is worse then the original)

- When applying the log scale the PVC is not corresponding to the distortion. The PVC must be rescaled to the log scale.

- When applying: 1) Zoom All 2) Log Scale 3) Lin Scale, the sonogram do not goes back to the linear scale

- After using ‘Zoomx2’ or ‘Zoomx0.5’ (without activating the ‘Zoom’ button) the zooming is left active (without any notification). It is different when using ‘ZoomAll’ (without activating the ‘Zoom’ button).

- When working with different spectrogram resolution (i.e. 64) the vertical slider with high zoom do not work properly.

- Does the sonogram presentation with the boxcar window is correct ?

- The ‘Spectral-Domain PVC Detector’ is not working properly. The determined tacks do not correspond with the chosen processing area if the zooming is active.

- The plug-in crashes when repeatedly applying the ‘Spectral-Domain PVC Detector’ with the following parameters:

- The error message in the  ‘Spectral-Domain PVC Detector’ must be given in English. See the next picture.

- When applying the ‘High-Frequency Pilote Tone Tracker’ the algorithm should first look for a pilot tone in the PSD spectrum, and it should start processing only when some kind of a pilot tone can be found. Or a proper info should be given to the user: ‘ No pilot tone was found. Should I start processing ?’. Currently the algorithm can be blindly applied to each sound sample.

- Some debugging is necessary. For example the plug-in crashes when applying the ‘High-Frequency Pilote Tone Tracker’ on the MTS examples twice with the parameters set as shown in the following pictures (concern both of the tested MTS sound samples). This do not happens when the bias sample is processed.

- Plug-in crashes when applying: 1) ‘Power-Line Hum Frequency Tracking, 2) ‘Show Hum’, 3) ‘Log Scale’.

- The instruction must be updated with the new options, i.e. zooming, log scale, others ...

New problems:

- Plug-in crashes when applying: 1) Change the resolution to 64 2) apply the log scale.

- When using the ‘Spectral-Domain PVC Detector’ and activating zoom, the ‘Create PVC’ button stays at the interface as a leftover form the algorithm menu (see the picture below)

- Conclusions and further work directions

The encountered problems must be solved. Then the new version of the plug-in must be tested again.

- Appendix

- P. Maziewski, “DeWower plug-in tests”, Internal PrestoSpace Technical Report No. 02/2007, GUT, MSD, (16 October 2007).

- M. Szczodrak, “Testing and implementing new features of DeWower VST plug-in”,  Internal PrestoSpace Technical Report No. 02/2007, GUT, MSD, (16 October 2007).