WA Restoration

‘ANR’ VST plug-in manual

DATE	December 7th 2007

AUTHOR, COMPANY	Michał Papaj (GUT)

KEYWORDS	Noise reduction, whitening, spectral subtraction.

RELATED ITEMS	Additive noise, forming filter.

DOCUMENT HISTORY

| Release | Date | Reason of change | Status | Distribution |
| --- | --- | --- | --- | --- |
| 1.0 | 07.12.2007 | First Draft | Living | For the internal use only |

Contents

1	Introduction	3

2	First run preparations	4

3	Algorithms of noise detection and reduction	5

3.1	Detection of noise	6

3.2	Noise reduction algorithm	7

4	Plug-in interface	8

4.1	Algorithm dependent parameters	9

4.1.1	Frame length	9

4.1.2	Segment length	9

4.1.3	Segment overlap	9

4.1.4	Noise detection threshold	9

4.2	Plug-in dependent parameters	10

4.2.1	File replacing	10

5	Noise reduction scenarios	11

# Introduction

The archival recordings stored on magnetic tapes are usually distorted with the additive noise. In this paper we present the tool that is design to reduce such a noise using statistical and digital signal processing algorithms for detection and denoising the recording.

The first problem that is met when working on noise reduction in archival recordings is identifying the noise only recording’s segments, which allows us gathering the noise profile necessary for further processing. The ANR plug-in uses statistical methods for indicating the segments which are most probably noisy, then, using the comparative criteria the proper segments are selected and processed. After successfully detecting the noise we can follow to main problem of our work – reduction of additive noise. Generally the algorithm of noise reduction implemented in the ANR plug-in is based on a frequency processing using signal segmentation and adaptive filtering methods. Below the example of noise reduction of the archival recording is presented.

Fig. 1 Signal before (upper) and after (lower) reduction

In the next sections we introduce installation procedures, plug-in parameters available for users and several scenarios showing usage of the ANR plug-in.

# First run preparations

The ANR plug-in is based on the VST environment thus it is installed as an additional library for host applications such as: WaveLab, FL Studio, Sound Forge  (further on we will use WaveLab as a reference VST host). As the ANR plug-in is using off-line processing procedures it is vital that the host supports VST off-line processing (plug-in is written with VST v 2.3 specification). Installation procedure is simple and requires copying ANR.dll file into the folder that is indicated as VST plug-ins container. After restarting the host progam we should be able to choose the plug-in from the off-line plug-ins list (Fig. 2).

There might be necessity of installing Microsoft .Net Framework in order to use ANR plug-in

# Algorithms of noise detection and reduction

In the following section the main idea and scheme (Fig. 3) of the ANR algorithm is presented. The automatic noise reduction uses noise detection algorithm which is based on two statistical features that are not available for user to interfere with and the decision algorithm based on comparison. The first operation is segmentation of signal to the frames (length of the frame is available to set, default value is 96000 Sa) with a proper overlap. Next the current frame is loaded into memory and once more is segmented into short segments (default 4800 Sa). The segments are than check for the signal presence. As there is found segment consisting noise only the whole frame is being processed (denoised). On the opposite, if the noise segment cannot be found in entire frame, the frame is buffered, and denoising is performed as soon as the noise profile is obtained. The process of noise reduction starts with adaptive whitening, where current segments are filtered accordingly to the changes in segments spectrum. The second step of denoising is spectral subtraction of the noise profile for the current frame. The signal frame is then reconstructed and given to the output.

## Detection of noise

## Noise reduction algorithm

# Plug-in interface

In the figure below the interface of the ANR plug-in is presented. It allows user to define 5 parameters. Four of them give the control on the noise reduction algorithms while the 5th defines the way the original file is treated by plug-in. All parameters are described in next two sections.

## Algorithm dependent parameters

### Frame length

The frame length defines the length (in samples) of the part of the file which is loaded into memory. The frame length must be much higher then the segment length in order to increase chances of finding the noise profile in the current frame, however, setting it too low or too high should not have any negative effects. As was mentioned in Sec. 3 each frame is overlaped, but the length of the overlap is unavailable for user to set. The reason of hiding the overlap parameter is that changing its value (in proper range) may only influent on processing time, while setting it to low may cause distortions in output file.

### Segment length

The length of segment represents also the length of noise profile that is necessary for the noise reduction. As the speech or music can be modelled as a stationary process only in a short period of time it is vital that the value of this parameter is not set to high. Second problem with this value is that the silence (part of signal where only noise with no speech or music appears) can be really short and setting the high value of segment length may cause finding the profile impossible. On the other hand setting this parameter too low makes the noise profile unrepresentative and may weaker the effect of noise reduction. The default value of segment length should be set to 4800 Sa.

### Segment overlap

Segment overlap is the next parameter given to the user under control. It influences not only the processing time (the higher overlap, the longer processing) but it also protects signal from clicks and crackles appearances while reconstructing frames.

### Noise detection threshold

Noise detection threshold is the last parameter that is available for user to set in process of noise detection. It indicates the level of relative spectrum flatness of segment. The higher value is set the more flat the noise spectrum must be.

## Plug-in dependent parameters

### File replacing

# Noise reduction scenarios

Several scenarios of noise reduction are fully presented. The step by step instructions and the results discussion are included. Different parameters sets for various sound samples are shown to cover full spectrum of possible scenarios that can be met.

### Noise reduction – scenario 1

### Noise reduction – scenario 2

### Iterative noise reduction – scenario 3

Let assume that we already denoised the sound sample (like we did it in scenario 2), however it seems that the noise can be reduced more than we obtained. We can try using the algorithm once more, and process the sample more precisely.

Each processing of sound sample involves attenuation of sound, thus at the end we can amplify the output sample: