Drift, Wow and Flutter Measurement and Reduction in Shrunken Movie Soundtracks.

P. Maziewski, A. Kupryjanow, A. Czyzewski

Abstract

Paper presents a set of algorithms for determining and reducing drift, wow and flutter (hereafter wow) in shrunken movie tapes. The idea behind the algorithms is to use image processing for calculating a local tape shrinkage which is one of the reasons for wow. Shrinkage can be calculated via analyzing the height of: a movie frame, sprocket hole, pitch or another standardized movie tape element; and then it can be expressed as a wow characteristic. After wow determination both the soundtrack and movie frames can be corrected. As wow reduction can be applied for the images and digital sound, algorithms for translating soundtrack images to digital waveforms are also given.

Précis

Most of films suffer from tape shrinkage. It is caused by loss of water, solvent and plasticizer in movie tapes. Both nitro based and acetate based films are exposed to this process. Only a fresh tape taken from the production line meets its normative requirements whereas the archive films all have different shrinkages. Such tape deformations are one of the reasons for wow distortions in the corresponding soundtrack. Additionally, shrinkage affects not only audio but also pictures, i.e. frames proportions are disturbed. In most cases tapes are uniformly shrunken, however, in some situations irregular deformations are possible. Those lead to accidental wow distortions which are highly perceptible and unpleasant. Thus, having a procedure for determining and reducing such distortions is of high importance.

The proposed paper presents a set of algorithms for determining and reducing audible and visible effects of movie tapes shrinkage. The algorithms can be used separately, e.g. only for wow reduction, however, the most convenient way is to use them in the following combination, which allows for the simultaneous sound and image restoration:

Figure 1. Algorithms set up for reducing audible and visible effects of movie tapes shrinkage.

The proposed processing is based on a frame-by-frame scheme. First the movie tape is scanned manually or automatically. Automatic scanning can be applied for tapes in good condition, i.e. mostly modern ones, whereas fragile and damaged archive films must be digitized manually. Next some initial preprocessing, i.e. contrast adjustments, mathematical morphology or other image processing techniques, can be applied to facilitate further processing. Wow characteristic determination (called the Pitch Variation Curve – PVC) is performed using the image features analysis such as the height of: a movie frame, sprocket hole or pitch. Consecutive heights - normalized using their standardized values - constitute the PVC. Based on the determined wow characteristic, the soundtrack images as well as the movie frames images can be corrected to compensate the shrinkage. Two-dimensional nonuniform resampling is used to obtain the restored pictures. The wow reduction can be also applied for the corresponding digital audio. Then it only requires an one-dimensional nonuniform resampling, which leads to a shorter computational time. For digital audio processing, algorithms for image to digital sound translation are needed. Thus they are given in the paper. At the end, the corrected sound and pictures can be further processed using some external tools and finally both can be combined into the restored digital movie.

The paper presents the theoretical study behind the proposed algorithms as well as the experiments which verify their performance. Theory reveals the connection between the newly proposed approach and the known techniques for wow determination and reduction. Experiments are focused on proving the theoretically evaluated performance. They involve genuine and artificially simulated tape shrinkages for the 16mm and 35mm films. Simulations are necessary to examine the maximal wow modulation frequency which can be captured in the determined PVC. All of the presented experiments prove the idea behind the proposed approach.

Paper outline will be as follows:

1. Introduction (PM)

Will cover: a short description of drift, wow and flutter phenomena (wow); description of movie tape shrinkage – causes and effects – as the reason for wow in movie tapes; overview of known approaches to wow determination and reduction, i.e. review of art; and the final conclusion, i.e. wow in shrunken movie tapes can be reduced differently with the usage of simultaneous sound and image processing.

2. Concept of Simultaneous Sound and Image Restoration (PM, AK)

Short introduction will present the main idea behind the proposed approach, i.e. wow characteristic can be determined using the image features analysis such as the height of: a movie frame, sprocket hole or pitch. Here the main processing scheme will be presented (Figure 1). Further, each processing block will be described separately. Scanning, however, will be include also in the introduction.

2.1 Image Preprocessing (AK)

All the preprocessing procedures will be described here. Those are: picture rotations, techniques for image enhancement, i.e. contrast adjustments or mathematical morphology, soundtrack and movie frame pictures extraction. Pictures illustrating all of the procedures will be also included.

2.2 Pitch Variation Curve Calculation (PM)

Here the concept of the PVC calculations using image features analysis will be presented in details. Also the theoretical constrains on the maximal modulation frequency which can be captured in the PVC will be presented. Experiments in the 3rd section will base on the theoretically determined values.

2.3 Image to Sound Translation (AK)

Here the concept/method of translating images of optical soundtrack to digital sound will be presented in details. Some of the experiments presented in the 3rd section will base on this findings.

2.3.1 A-Chain (AK)

Here the A-Chain characteristic for filtering film soundtracks will be presented. The digital filter which simulates this characteristic will be described.

2.3.2 Wow Reduction (PM)

Here the wow reduction for digital sound will be presented.

2.4 Movie Frames Correction (AK, PM)

Here the wow reduction for digital scans of movie frames will be presented. Algorithm for movie frames cutting from scans and joining them to movie file will be described.

2.5 Combining Sound with Image and Additional Processing (AK)

Description of sound and image combining.

3. Experiments (PM, AK)

The experiments aim at proving the proposed concept of image-based PVC determination and wow reduction. They involve genuine and artificially simulated tape shrinkages for the 16mm and 35mm films. In particular the maximal modulation frequency will be evaluated and compared with the theoretical findings. Genuine tape shrinkages will be also presented (based on the materials found in MSD repositories).

3.1 Picture to Sound Translations

Will include an example of a digital sound translated from high resolution images compared against the same soundtrack recorded using a movie projector. Comparison will include two scenarios: with and without the A-Chain filtering characteristic simulation.

3.1 Maximal Modulation Frequency Constrain (PM)

Here the maximal modulation frequency simulations results will be presented.

3.3 Processing Examples (PM, AK)

Here the examples of  real-life tape shrinkage found in MSD repositories will be presented as well as the restoration results.

3.1 16mm film

3.2 35mm film

4. Summary (PM)

5. References (PM, AK)

2.2 Pitch Variation Curve Calculation (PM)

In the wow processing algorithms [JAES] the maximal modulation frequency (FMmax) which can be captured in the determined PVC is set by two factors [AA]: the Nyquist sampling theorem and quasi-stationarity criteria. The first one can be express as:

FMmax<0.5min(Fswow)					(1)

where Fswow is the nonuniform (due to tape shrinkage) sampling frequency of the PVC. In order to asses the FMmax following assumption can be made:

min(Fswow)Fs					(2)

where Fswow represents the uniform (for non-shrunken tapes) sampling frequency of the PVC. Such assumption is true when the local tape shrinkage occurs. Then the width of tape elements is lower causing a smaller distance between them. As the result the Fswow, which is set by the elements distance in a shrunken movie tape, is always higher then the Fs for the even tape. Thus the eq.1 can be express as:

FMmax< 0.5Fs						(3)

The quasi-stationarity criteria comes from the assumption that the modulation frequency should be stable within the period of time Thwow which corresponds to the height of the analyzed movie tape element (i.e. movie frame, perforation or pitch) played with a standard tape reproduction velocity. The criteria can be expressed in a form of equation:

FMmax« min(1/Thwow)						(4)

As the frequency equivalent to the Thwow is always higher than the Fs – due to the simple facts that Thwow does not exceed the 1/Fs (look at the figure no. ?) and the elements shrinkage lowers the Thwow, assuming the same standard tape reproduction velocity – eq. 4 can be express as:

FMmax« Fs						(5)

In the following FMmax was asses as 10 times smaller than the Fs, which satisfies both the Nyquist theorem (eq. 3) and the quasi-stationarity criteria (eq. 5).

When analyzing the height of a movie frame the Fs equals the number of frames pre second (fps). This mean that for each frame one PVC value can be calculated. Different Fs values come from analyzing the perforation of pitch. Then the Fs equals the number of  perforation elements which are placed in the movie tape within 1 second length (pitch per second - pps).

Replacing the Fs with the standardized fps and pps values in eq. 5 allows to calculate FMmax border values:

Tab.1 FMmax limitations [MovRes] [Mal53].

| Film format [mm] | 8 | 16 | 35 | 65/70 |
| --- | --- | --- | --- | --- |
| frame per sec | 16[Wiki] | 24[Wiki] | 24[ANSI] | 24[Wiki] |
| FMmax [Hz] | 1.6 | 2.4 | 2.4 | 2.4 |
| perforations per sec | 16[Wiki] | 24[MovRes] | 96[ANSI] | 120[Wiki] |
| FMmax [Hz] | 1.6 | 2.4 | 9.6 | 36 |

Empirical evaluation of the determined maximal modulation frequency will involve wow simulation in movie tape images for 0.5FMmax,  FMmax , 2FMmax values and two deviation values: 0.1 and 0.01.  Then wow characteristic determination will be apply and at the end both simulated and determined PVCs will be compared.

3.1 Maximal Modulation Frequency Constrain (PM)

Experiments verifying the maximal modulation frequency constrain were performed as follows. For a given 35 mm film its tape scans (taken frame-by-frame) were resampled in order to simulated wow distortions. The simulated modulation frequency (fm) was set to 0.5FMmax,  FMmax , 2FMmax, i.e. 1.2 Hz, 2.4 Hz and 4.8 Hz. The relative frequency deviation – defined as the ∆f/fm  - was set to 0.1 and 0.01 for simulating more and less noticeable wow distortions. Next, the modulated pictures were processed by the wow determination algorithm and the resulting PVCs were compared against known FM patterns. The results are are given in the following pictures:

Figure e1. Wow determination results for the higher relative deviation (0.1). The FM values from the left side are: 1.2, 2.4 and 4.8 Hz.

Figure e2. Wow determination results for the lower relative deviation (0.01). The FM values from the left side are: 1.2, 2.4 and 4.8 Hz.

Figure e3. Tape shrinkage example taken form a 35 mm film with a variable area soundtrack.

5. References

[JAES] A. Czyzewski, A. Ciarkowski, A. Kaczmarek, J. Kotus, M. Kulesza, P. Maziewski, ”DSP Techniques for Determining “Wow” Distortion,” in Journal of the Audio Engineering Society 55, No. 4, pp. 266-284 (April 2007).

[AA] P. Maziewski, ”Evaluation of the Maximal Modulation Frequency for Wow and Flutter Determination,” Proceedings of 12th International Symposium on Sound and Vision Engineering and Mastering, Gdansk, Poland, (June 15-16, 2007).

[MovRes] P. Read, M. P. Meyer, Restoration of Motion Picture Film, Oxford: Butterworth-Heinemann 2000.

[Mal53] I. Malecki, Technika Nagrywania i Odtwarzania Dźwięków, Warszawa: PWT 1953.

[ANSI] ANSI/SMPTE 102-1997. SMPTE STANDARD for Motion-Picture Film (35mm) - Perforated CS-1870. Society of Motion Picture and Television Engineers. White Plains, NY.

[Wiki] Wikipedia based – must be replaced by a proper references.

[Maz] MAZIEWSKI P., SZCZUKO P. : System Synchronizacji nagrań wielokanałowych, Zeszyty Naukowe Wydziału Elektrotechniki i Automatyki Politechniki Gdańskiej Nr.19, Seria: Zastosowanie  Komputerów w Nauce i Technice 2003, str. 79 - 84, Gdańsk 2003.

[Urb] URBAŃSKI B.: Technika Zapisywania i Odczytywania Dźwięków, wyd. kom. i łącz.,

Warszawa 1978