Title: Reduction of Parasitic Pitch Variations in Ethnic Music Audio Documents.

Authors: Andrzej Czyzewski, Przemyslaw Maziewski, Adam Kupryjanow

Affiliation: Gdansk University of Technology, Multimedia Systems Department, ul. Narutowicza 11/12, 80-226 Gdansk, POLAND; {andcz, przemas, adamq}@sound.eti.pg.gda.pl, tel. +48 58 3471301, fax +48 58 3471114.

Abstract: This paper presents the new method for reducing the parasitic pitch variations in archival audio recordings. The new method is intended for analyzing the movie soundtracks recorded in optical films. It utilizes image processing for calculating and reducing the effects of tape shrinkage which is one of the main reasons for parasitic pitch variation in film audio. As long as the film tape characteristics are known the new method can be easily tuned to analyzed obsolete, non-professional archival recordings. In the paper the new method is compared with the previous approaches.

Keywords: audio restoration, parasitic pitch variations.

list of unusual symbols used in the article

number of pages, tables and figures

1. Introduction

The purpose of this work is to present a new method for reducing parasitic pitch variations in archival audio recordings and compare it with known approaches. Parasitic pitch variations, mainly wow and flutter, can be defined as unwanted frequency modulations added to the signal during recording, producing, monitoring or duplicating phases. Wow and flutter are defined based on the range of the parasitic modulation frequency fm: wow fm ∈〈0.5, 6 Hz), flutter  fm∈〈6, 100 Hz〉 [AES stand]. The distortions are introduced to the signal by the irregular velocity of its analogue carrier. The irregularities can originate from various mechanisms, depending on the carrier type, its condition and damages, its production technique, and other factors [1JAES]–[3JAES]. The irregularities are commonly found in ethnic music recordings made by an unskilled staff using nonprofessional recording systems. Especially in eastern Europe low quality of the audio carriers, poor alignment and maintenance of the recording and reproducing equipment enhanced the risk of wow and flutter in ethnic audio recordings. Example of a badly produced movie tape splice, taken from a Polish archive film – is given in fig. 1. When such a tape makes a contact with the projector (or telecine) sprocket roller it moves slightly back or towards its original movement direction, depending on the difference between the interval of successive perforation holes and the distance between the teeth of the transpiration system (see fig. 2). Both cases result in a momentary pitch modulation of the soundtrack. The other reason for wow and flutter in movie ethnic movie soundtracks is the tape shrinkage. It is caused by loss of water, solvent and plasticizer in movie tapes. Both nitro based and acetate based films are exposed to this process. Movie tape shrinkage causes similar problems as the ones presented with the badly produced splice. As the shrink perforation do not match the distances between the teethes of the sprocket roller, it causes movie tape shifts each time a transportation teeth enters the perforation hole. This is presented in fig. 2. Different reasons for wow and flutter can be found in magnetic and mechanical recordings [JAES]. Again most of them are triggered by a poor equipment or lack of appropriate production standards, thus are highly susceptible in ethnic recordings. The authors of this papers presented methods for correcting wow and flutter in the audio recordings. They utilize some ancillary information retrievable from the audio documents. i.e. initial frequencies of the power line hum, high frequency bias, NTSC pilot tone. Short overview of those will be presented in the next section and the details can be found in the literature [JAES]. New approach was recently studied by the authors [AESCon]. It is dedicated to movie soundtracks and utilizes image processing of the movie tape where the ancillary historical information like tape format, its initial speed, recording  procedure play crucial role. After providing those information, which can be very unique for some of the obsolete recordings, the reduction of the pitch variations of the movie soundtrack can be greatly improved and even done automatically for tapes with a good condition.

2. State of the Art (PM) 2 str

- Gerzon (idea on hfb, no implementation)

The ideas of algorithms for wow and flutter determination and reduction were introduced by Gerzon [Gerzon]. He suggested using the leftover of the high-frequency bias, which can be found in magnetic tape recordings, as the pilot tone depicting the distortions’ course. Additionally, he proposed the nonuniform resampling technique  for reducing the distortions. However, as Gerzon did not present any DSP algorithms, he did not verified his ideas.

[Gerzon] M. Gerzon, “Don’t Destroy the Archives!,” unpublished (1992); available at www.waves.com/german/

htmls/service/faq/dont.html (accessed 2006 Oct.).

- Godsill and Nicholson (first implementation but with problems)

First DSP algorithms for wow course determination and reduction were presented by Godsill [Godsill 1]. Then, further developed, they were reported by him with several co-researchers [Godsill 2 – 5]. The algorithms aimed at  automatic correction of smooth pitch variations over long time scales. They were build on a three-step processing scheme. First the preprocessing, i.e. standard sinusoidal modelling analysis, provided information on the frequency partials. Second the partials were analyzed in order to determine the wow course. Third nonuniform resampling was used for the distortion reduction. The main emphasis was put on the second step, i.e. the determination of the wow course. Here different a priori models of the distortion course, from simple sinusoidal to more complex regressive processes, were examined using the Bayes’ estimation scheme. It was thru those models that the ancillary information about the recording medium, gramophone records in this case, could be used.

Partials’ processing was also used by Nichols in his approach to the wow determination and reduction [Nichols 1] [Nichols 2]. First of his algorithms was similar to the Gosill’s approach with only minor improvements added, such as the spectrum warping or the iterative procedure for improving the frequency estimation. Second algorithm was build on a novel concept of graphical processing of the spectrogram. Herein partials were build using image processing routines used to select the pixels representing the tonal components from the spectrogram. In both algorithms, partials were identically  processed in order to determine the wow course. As the algorithms designed by Nichols were dedicated to phonograph recordings restorations, a simple sinusoidal model was used to describe wow. The non-uniform resampling was then applied to reduce wow.

he introduced the iterative procedure for a better estimation of the spectral peaks’ frequency as well as some additional features,

such as spectrum warping

Similar approach to A similar approach to wow processing was studied by Nichols [11], [12]. He presented the iterative application J_PITCH with two algorithms for wow determination (and an additional one for drift processing) and the nonuniform resampler for the final restoration. The first of Nichols’ algorithms for wow determination was similar to Godsill’s concept of partial processing. Nichols, however, introduced the iterative procedure for a better estimation of the spectral peaks’ frequency as well as some additional features,

such as spectrum warping. The second algorithm was based on a novel concept of graphical processing of

the spectrogram. The time–frequency spectrogram was treated as a two dimensional graphical object. It was then

searched for peaks, which, after the preprocessing necessary to exclude some false elements, were joined to form

partials. Then the partials were postprocessed in the same manner as those from the first algorithm.

Godsill’s method suffered from some drawbacks. It addresses only smooth and repetitive pitch variations over long time scales, mainly wow, whereas the real-life wow and flutter distortions are frequently unique and short in time. It was complicated, that is, it requires choosing and

tuning the distortion model, which can be problematic for nonspecialists. Moreover it addresses only smooth and

repetitive pitch variations over long time scales whereas the real-life wow distortion is frequently unique and

short in time. Furthermore, some open questions regarding the method remain, such as the preprocessing influence of sinusoidal modeling on the results as well as the vibrato versus wow discrimination.

It was also explored by Nichols [11], [12].

[Godsill 1] S. J. Godsill, “The Restoration of Degraded Audio Signals,” Ph.D. dissertation, Cambridge University, Cambridge, UK (1993).

[Godsill 2] S. J. Godsill and P. Rayner, “The Restoration of Pitch Variation Defects in Gramophone Recordings,” in Proc. IEEE Workshop on Applications of Signal Processing to Audio and Acoustics (Mohonk, NY, 1993 Oct. 17–20), pp. 148–151.

[Godsill 3] S. J. Godsill, “Recursive Restoration of Pitch Variation Defects in Musical Recordings,” in Proc. IEEE Int. Conf. on Acoustics, Speech and Signal Processing 2, (Adelaide, Australia (1994 Apr. 19–22), pp. 233–236.

[Godsill 4] S. J. Godsill and P. J. W. Rayner, “Restoration of Pitch Variation Defects,” in Digital Audio Restoration—A Statistical Model-Based Approach (Springer, London, 1998), pp. 171–190.

[Godsill 5] S. J. Godsill, G. Reid, and C. Hicks, “Restoration of Smooth Pitch Variations over Long Timescales,”  presented at the Joint Technical Symposium, Toronto, Canada (2004 June 24–26); available at http://www.jts2004.org/english/proceedings/Reid.htm (accessed 2006 Oct.).

[Nichols 1] J. Nichols, “An Interactive Pitch Defect Correction System for Archival Audio,” in Proc. AES 20th Int. Conf. (Budapest, Hungary, 2001 Oct. 5–7), paper 1941.

[Nichols 2] J. Nichols, “High Performance, Low-Cost Wax Cylinder Transcription System,” in Proc. AES 20th Int. Conf. (Budapest, Hungary, 2001 Oct. 5–7), paper 1937.

- Wolfe and Howart (reported implementation but no details)

Howart and Wolfe reported an algorithms for wow and flutter determination and removal [H&W]. They designed a system being a combination of the hardware part for capturing the high frequency bias and the software part for wow and flutter reduction. Thus their desing follows the concept proposed by Gerzon. Additionally, those authors presented also the study of the non-uniform resampling technique used for wow and flutter reduction [W&H].

[W&H] P. Wolfe and J. Howarth, “Nonuniform Sampling Theory in Audio Signal Processing,” presented at the 116 Convention of the Audio Engineering Society, J. Audio Eng. Soc. (Abstracts), vol. 52, p. 813 (2004 July/Aug.), convention paper 6123.

[H&W] J. Howarth and P. Wolfe, “Correction of Wow and Flutter Effects in Analog Tape Transfers,” presented at the 117 Convention of the Audio Engineering Society, J. Audio Eng. Soc. (Abstracts), vol. 53, p. 91 (2005 Jan./Feb.), convention paper 6213.

- Our approach (reported on DSP algorithms)

Wow and flutter determination and reduction was studied also by the authors of this paper. Different algorithms were proposed for wow course determination such as [JAES]: the power line hum tracker, high frequency bias tracker which was also successfully used for the Multichannel Television Sound (MTS) pilot tone tracking [Pastuszak], the spectrum centre of gravity analyser. The algorithms were tested on real-life archive recordings [JAES]. Simulation were also made in order to determined their limitation in terms of the maximal modulation frequency [Maziewski 2]. It was found that only the high frequency bias tracker can determined both wow and flutter courses, whereas the other algorithms can process flutter only to some extent. We have also studied the non-uniform resampling influence on the quality of the restored recordings [Maziewski 1]

[JAES] A. Czyżewski, A. Ciarkowski, A. Kaczmarek, J. Kotus, M. Kulesza, P. Maziewski, DSP Techniques for Determining “Wow” Distortion, „Journal of the Audio Engineering Society” April 2007, vol. 55, nr 4, str. 266-284.

[Pastuszak] P. Pastuszak, MTS Pilote Tone Tracking for “Wow” Determination, Archives of Acoustics 2008, vol. 33, nr 1, str. 117-123.

[Maziewski 1] P. Maziewski, Wow defect reduction based on interpolation techniques, „Bulletin of the Polish Academy of Science: Technical Sciences” 2006, vol. 54, nr 4, str. 469-477. Dostępne online pod adresem: http://bulletin.pan.pl/(54-4)469.pdf (przeglądano w marcu 2008).

[Maziewski 2] P. Maziewski, Modulation Frequency Constrains on Wow and Flutter Determination, Archives of Acoustics 2008, vol. 33, nr 1, str. 125-131.

- Vitaliy Fadeyev, Carl Haber (audio restoration image based for mechanical – grove-based – recordings)

- E. Brun1, A. Hassa¨ıne1, B. Besserer2, E. Decenci`ere1 (DSP metdhs for soundtracks restoratrion)

- Detlef Richter Dieter Poetsch Andy Kuiper (DSP methods for soundtrack restoration)

All of the presented algorithms for wow and flutter correction were based on a similar idea: the audio documents contents, either the audio itself or additional signals like hum or bias,  were analyzed in order to determine the distortion course. Meanwhile a different approach can be imagined: the analysis of the audio carrier. Some interesting work already exists. Fedeyev and Haber proposed image processing of the mechanically made recordings (modulated groves) [F&H].  In their approach the grove shapes are being scanned and translated into sound. Image analysis and correction allows to virtually repair carrier damages, which further leads to audio restoration. I.e. such processing allows for wow and flutter reduction which was triggered by the ellipticity or eccentricity of the recording.  Sounds from other carriers can be similarly restore. It was reported that image processing can be used for movie soundtrack restoration [B&H&B&D], [R&P&K].

Image processing of the optical tapes scan can be used for wow and flutter determination reduction in movie soundtracks. The algorithms presented in the next section allows to determined the wow and flutter (to some extend) course.

[F&H] V. Fadeyev, C. Haber, A novel application of high energy physics technology to the problem of audio preservation, Nuclear Instruments and Methods in Physics Research A 518 (2004) 456–462.

[B&H&B&D] E. Brun, A. Hassaine, B. Besserer, E. Decenciere1, Restoration of Variable Area Soundtracks, IEEE International Conference on Image Processing, ICIP 2007, 16-19 September 2007, San Antonio, Texas, USA

[R&P&K] D. Richter, D. Poetsch, A, Kuiper, Localization of Faults in Multiple Double Sided Variable Area Code Sound Tracks on Motion Picture Films Using Digital Image Processing, in 13th International Czech - Slovak Scientific Conference Radioelektronika, 2003.

Overview of the DSP-based methods for wow and flutter reduction (reference-based, i.e. JAES). It should be indicated here that some of the methods already utilize “the ancillary information, retrievable from the history of the audio document and from the history of the musical technology connected with it” (quotation from the “aim of the special issue”). Those are the “pilot tone analysis” methods. However, those are not always applicable, i.e. special features are needed like the high frequency bias. On the other hand methods which rely only on audio analysis can produce false results, e.g. wow vs. vibrato discrimination. Incorrect results are highly susceptible when analysing unique ethnic audio documents which have no reference scores or similar performances to camper with. In such situation the new approach i.e. audio carrier analysis, allows to overcome some of the reported problems.

3. New Approach (AK) 2 str

The idea behind the algorithms is to use image processing for calculating the local tape shrinkage which is one of the reasons for drift, wow and flutter. The shrinkage can be calculated via analyzing the image height of: a movie frame, sprocket hole, pitch or another standardized movie tape element; and then it can be expressed as the drift, wow and flutter characteristic. After the characteristic determination both the soundtrack and movie frames can be corrected.

This methods can be used to reconstruct soundtracks from 35mm and 16mm film with optical soundtracks. In films where sound was recorded at magnetic soundtrack only image of movie frame can be corrected. It is expected that movie frames will be digitalize properly with high spatial resolution and at least  48 bits colour depth. Properly scanned images means that the image of movie frame is parallel to  edge of scanned image and sharpness of picture is correct.

One PVC value can be calculated for each movie frame. This value is defined as the local tape shrinkage which affects this particular frame. For the first frame, it can be expressed as:

PVC(t1)=Fheigth1Fheigthnom	(1)

where the Fheigthnom represents the nominal frame height (for a standard tape without any shrinkage) and. the time point t1 can be expressed in seconds as:

t1=Fheigth12H1sec	(2)

where the H1sec represents the movie tape height of a one second film  recording.

Successive PVC values can be calculated as:

PVC(tn)=FheigthnFheigthnom	(3)

Similar to eq. 2the time point tn can be expressed in seconds as:

tn=m=1n−1Fheigthm+Fheigthn2H1sec	(4)

In the next section the algorithm for frames (or perforation) height calculation is presented.

3.1 Frame height calculation

To calculate frame height we have to know how movie frames looks. There are a little differences between frames in 35mm and 16mm films. In 35mm films there is black area between two frames as it is shown in figure XXX1 and frame in 16mm films are located close to each other without any black area. Although the same image processing can be used for 35mm and 16mm films.

To determine size of movie frame there is simple edge detection algorithm used. It is possible to used this  algorithm because the shape of elements that are measured is regular and not complicated . So only location of two parallel lines corresponding to frame beginning and end located perpendicular to  soundtrack image have to be find.

In this method of PVC determination  the image of the movie frame contains whole current processed frame and a part of frame before and behind it. It is necessary to have data redundancy to see where image of current movie frame begins and ends. To calculate frame height image of film is cut and decimal-to-binary conversion is made. This operation haul up edges. Then mean value of every row of image pixels is calculated. Analyzing  values of the mean signal helps to determine beginning and the end of two contrast areas. To do this more accurately from the mean signal  derivate is calculated and instead of mean signal analyzing, derivative of it is processed. As it is shown at the figure XXX2 beginning and the end of the frame corresponds to local maximal values of the derivate signal.

3.1 Algorithm for calculating height of tape perforation

This method of PVC determination give improvement of quality only for 35mm films, because at this type of films one move frame  corresponds to four perforation holes and at 16mm films one movie frame corresponds only to one perforation hole . So usage of this algorithm for  35mm film allows to determine four times higher modulation frequency than in algorithm that determine PVC from movie frame height. Way of  finding beginning and end of  perforation holes is similar to height of movie frames calculation. In this algorithm images of film tape contains only image of current processed movie frame and image of perforation holes that corresponds to this frame.

From whole movie frame image perforation image is cut and perforation size is calculated from it. To determine size of holes decimal-to-binary conversion is made like in frame height calculation algorithm. Next mean value of image rows is calculated and from this signal derivate is taken. Beginning of the first perforation hole corresponds to the image beginning and beginning of the last  perforation holes corresponds to the image end. Other beginnings of  the perforation are calculated as local maxims of derivate from mean value signal. Height of perforation hole that is analyze in this PVC determination algorithm isn’t really a height of it but a distance between two beginnings of neighbouring perforation holes.

4. Experiments / comparison of the new method against known approaches

Experiments will cover modulation frequency range estimation for the new and the old algorithms. Estimated values will allow to compare the algorithms.

Table 1	Frame based maximal modulation frequency limitations [AESconv]

| Film format [mm] | 8 | 16 | 35 | 65/70 |
| --- | --- | --- | --- | --- |
| Tape velocity [fps] | 16 | 24 | 24 | 24 |
| fmMax[Hz] | <3.2 | <4.8 | <4.8 | <4.8 |

Table 2	Perforation based maximal modulation frequency limitations [AEScon]

| Film format [mm] | 8 | 16 | 35 | 65/70 |
| --- | --- | --- | --- | --- |
| Tape velocity [pps] | 16 | 24 | 96 | 120 |
| fmMax[Hz] | <3.2 | <4.8 | <19.2 | <72 |

Table 3. Maximal modulation frequency limitations for the presented wow and flutter tracking algorithms [AA]

| Algorithms and their settings | FMmax |
| --- | --- |
| Power line hum tracker; M=8 Sa; Fs=200 Hz; | FMmax< 12.5Hz |
| COG tracker; M=1024 Sa; Fs=44100 Hz; | FMmax< 21.5Hz |
| High-frequency bias tracker; M=1024 Sa; Fs=192000 Hz | FMmax< 93.75Hz |

Table 4. Mean Square Error of the PVC determined using the power line tracker, fn=50Hz, max_dev=1.1

| fm [Hz] | 0.5 | 6 | 12 | 19 | 38 |
| --- | --- | --- | --- | --- | --- |
| Mean Value | 1,48E-05 | 8,38E-05 | 2,04E-04 | 3,63E-04 | 5,03E-04 |
| Variance | 1,21202E-10 | 7,83447E-10 | 1,72818E-09 | 4,89267E-09 | 8,83871E-09 |
| Std. Deviation | 1,10092E-05 | 2,79901E-05 | 4,15713E-05 | 6,99476E-05 | 9,40144E-05 |

Table 5. Mean Square Error of the PVC determined using the power line tracker, fn=50Hz, max_dev=1.01

| fm [Hz] | 0.5 | 6 | 12 | 19 | 38 |
| --- | --- | --- | --- | --- | --- |
| Mean Value | 1,29E-05 | 1,36E-05 | 1,69E-05 | 1,96E-05 | 1,91E-05 |
| Variance | 7,71595E-11 | 6,47345E-11 | 4,98383E-11 | 5,04767E-11 | 4,48537E-11 |
| Std. Deviation | 8,78405E-06 | 8,04578E-06 | 7,05962E-06 | 7,1047E-06 | 6,69729E-06 |

Table 6. Mean Square Error of the PVC determined using the power line tracker, fn=60Hz, max_dev=1.1

| fm [Hz] | 0.5 | 6 | 12 | 19 | 38 |
| --- | --- | --- | --- | --- | --- |
| Mean Value | 1,69E-05 | 1,31E-04 | 2,61E-04 | 4,00E-04 | 5,27E-04 |
| Variance | 1,42213E-10 | 5,15422E-09 | 2,64248E-09 | 5,2909E-09 | 6,72049E-09 |
| Std. Deviation | 1,19253E-05 | 7,17929E-05 | 5,1405E-05 | 7,27386E-05 | 8,19786E-05 |

Table 7. Mean Square Error of the PVC determined using the power line tracker, fn=60Hz, max_dev=1.01

| fm [Hz] | 0.5 | 6 | 12 | 19 | 38 |
| --- | --- | --- | --- | --- | --- |
| Mean Value | 8,27E-06 | 8,69E-06 | 1,18E-05 | 1,24E-05 | 1,38E-05 |
| Variance | 3,88222E-11 | 3,18296E-11 | 3,49201E-11 | 3,01704E-11 | 2,61409E-11 |
| Std. Deviation | 6,23074E-06 | 5,64177E-06 | 5,90932E-06 | 5,49276E-06 | 5,11282E-06 |

Table 8. Mean Square Error of the PVC determined using the local tape shrinkage analysis, max_dev=1.1

| fm [Hz] | 0.5 | 6 | 12 | 19 | 38 |
| --- | --- | --- | --- | --- | --- |
| Mean Value |  | 1,66E-03 | 2,39E-03 | 2,53E-03 | 3,10E-03 |
| Variance |  | 7,77E-06 | 1,1E-05 | 1,15E-05 | 3,86E-05 |
| Std. Deviation |  | 0,002787 | 0,003322 | 0,003385 | 0,006216 |

Dokładność

Taśma 35mm, fm = 0,5 6 12 19 0,5 i 6 – jako granice wow, 12 – jako granica hum trackera, 19 jako wartość graniczna analizy skurczu dla tasmy 35 mm (perforation based) i 38 jako wartość dwa razy więszka

Symukacja kołysania i drżenia – jako złożony przebieg FM o maksymalnych wartościach FM. Dodatkowo wersja z szumem – powwinna zaniżyć dokładność analiz audio.

Nagranie dźwiękowe z

5. Conclusion 0,5 str

Will conclude the modulation frequency range comparison as well as the other advantages and disadvantages of the new method compared against the known approaches.

6. Summary 0,5 str

Acknowledgement

this work was supported by the Polish Government research resources for years 2007-2009 as the research project no. 4381/B/T02/2007/33.

Appendix

References

[AES] AES Std. 6-1982, Method for Measurement of Weighted Peak Flutter of Sound Recording and Reproducing Equipment,. Audio Engineering Society, New York, 2003.

Reference to a book:
[1] M. Kunt, Digital Signal Processing, Artech House, Norwood, MA, 1986, Chapter 6, pp. 319-338.

Reference to a journal publication:
[2] F.J. Harris, On the use of windows for harmonic analysis with the discrete Fourier transform, Proc. IEEE 66 (1) (1978) 53-83.

Reference to a conference paper:
[3] D. Coulon and D. Kayser, A supervised-learning technique to identify short natural language sentence, Proc. 3rd Internat. Joint Conf. on Pattern Recognition, Coronado, CA, 8-11 November 1976, pp. 85-89. 

Reference to a chapter in an edited book:
[4] E.F. Moore, The firing squad synchronization problem, in: E.F. Moore (Ed.), Sequential Machines, Selected Papers, Addison-Wesley, Reading, MA, 1964, pp. 213-214.

Vitae

Figures

Fig. 1 Example of badly produced movie tape splice

Fig. 2 Tape shrinkage effects on the movie tape transportation system [Malecki];

Fig. XXX1 Example of 35mm and 16mm films

Legends

Tables

Outline:

- Introduction (PM) 1str

Short introductory part indicating the purpose of this work, presenting basic definitions (i.e. parasitic pitch variations – mainly wow and flutter), examples of deformed movie tapes, mechanisms responsible for those distortions, known analogue approaches to the reduction of those distortions, etc.

- State of the Art (PM) 2 str

Overview of the DSP-based methods for wow and flutter reduction (reference-based, i.e. JAES). It should be indicated here that some of the methods already utilize “the ancillary information, retrievable from the history of the audio document and from the history of the musical technology connected with it” (quotation from the “aim of the special issue”). Those are the “pilot tone analysis” methods. However, those are not always applicable, i.e. special features are needed like the high frequency bias. On the other hand methods which rely only on audio analysis can produce false results, e.g. wow vs. vibrato discrimination. Incorrect results are highly susceptible when analysing unique ethnic audio documents which have no reference scores or similar performances to camper with. In such situation the new approach i.e. audio carrier analysis, allows to overcome some of the reported problems.

- New Approach (AK) 2 str

The idea behind the algorithms is to use image processing for calculating the local tape shrinkage which is one of the reasons for drift, wow and flutter. The shrinkage can be calculated via analyzing the image height of: a movie frame, sprocket hole, pitch or another standardized movie tape element; and then it can be expressed as the drift, wow and flutter characteristic. After the characteristic determination both the soundtrack and movie frames can be corrected.

- Experiments (PM, AK) 3 str

Experiments will cover modulation frequency range estimation for the new and the old algorithms. Estimated values will allow to compare the algorithms.

- Conclusion 0,5 str

Will conclude the modulation frequency range comparison as well as the other advantages and disadvantages of the new method compared against the known approaches.

- Summary 0,5 str

- References 1 str

Form: "Fast Communication", 10 pp, single column, double spaced.

Format:

- The manuscript must be written in English and the text of the paper should be preceded by an abstract of no more than 200 words.

- The first page should include the article title and the author's name and affiliation, as well as a name and mailing address to be used for correspondence and transmission of proofs.

- The second page should include a list of unusual symbols used in the article and the number of pages, tables and figures. It should also contain the keywords in English.

- A Fast Communication is a short, self-contained article not exceeding 10 pages (single column, double spaced) in length on ongoing research, or reporting interesting possibly tentative ideas, or comments on previously published research. The editorial decision is typically binary to provide rapid dissemination of the results. The objective is to provide detailed, constructive feedback on submitted papers and publish high quality papers within a very short period of time. The target for a first reply is two months.

- The text should be in single-column format

- do not use the wordprocessor's options to justify text or to hyphenate words

- Do not embed "graphically designed" equations or tables, but prepare these using the wordprocessor's facility

- When preparing tables, if you are using a table grid, use only one grid for each individual table and not a grid for each row. If no grid is used, use tabs, not spaces, to align columns. The electronic text should be prepared in a way very similar to that of conventional manuscripts

- Do not import the figures into the text file but, instead, indicate their approximate locations directly in the electronic text and on the manuscript