Title: Reduction of Parasitic Pitch Variations in Ethnic Music Audio Documents.

Authors: Andrzej Czyzewski, Przemyslaw Maziewski, Adam Kupryjanow

Affiliation: Gdansk University of Technology, Multimedia Systems Department, ul. Narutowicza 11/12, 80-226 Gdansk, POLAND; {andcz, przemas, adamq}@sound.eti.pg.gda.pl, tel. +48 58 3471301, fax +48 58 3471114.

Abstract: This paper presents the new method for reducing the parasitic pitch variations in archival audio recordings. The new method is intended for analyzing the movie soundtracks recorded in optical films. It utilizes image processing for calculating and reducing the effects of tape shrinkage which is one of the main reasons for parasitic pitch variation in film audio. As long as the film tape characteristics are known the new method can be easily tuned to analyzed obsolete, non-professional archival recordings. In the paper the new method is compared with the previous approaches.

Keywords: audio restoration, parasitic pitch variations.

List of unusual symbols used in the article:

fm - parasitic modulation frequency

fmax - maximal modulation frequency in the determined pitch variation curve

Fs - sampling frequency of a digitized audio recording

M - processing frame length

Number of:

pages - ?, tables - ? and figures - ?.

1. Introduction

The purpose of this work is to present a new method for reducing parasitic pitch variations in archival audio recordings and compare it with the known approaches. Parasitic pitch variations, mainly wow and flutter, can be defined as unwanted frequency modulations added to the original signal during recording, producing, monitoring or duplicating phases. Wow and flutter are defined based on the range of the parasitic modulation frequency (fm): wow fm ∈〈0.5 Hz, 6 Hz), flutter  fm ∈〈6 Hz, 100 Hz〉 [AES stand]. The distortions are introduced to the signal by the irregular velocity of its analogue carrier. The irregularities can originate from various mechanisms, depending on the carrier type, its condition and damages, its production technique, and other factors [1JAES], [2JAES], [3JAES]. The irregularities are commonly found in the ethnic music recordings made by an unskilled staff using nonprofessional recording systems. Especially in the eastern Europe low quality of the audio carriers, poor alignment and maintenance of the recording and reproducing equipment enhanced the risk of wow and flutter.

Example of a badly produced movie tape splice, taken from a Polish archive film – is given in fig. 1. When such a tape makes a contact with the projector’s sprocket roller (or telecine’s) it moves slightly back or towards its original movement direction, depending on the difference between the interval of successive perforation holes and the distance between the teeth of the transpiration system (see fig. 2). Both cases result in a momentary pitch variation of the soundtrack.

The other reason for wow and flutter in the ethnic movie soundtracks is the tape shrinkage. It is caused by loss of water, solvent and plasticizer in movie tapes. Both nitro based and acetate based films are exposed to this process. Movie tape shrinkage causes similar problems as the ones presented with the badly produced splice. As the shrink perforation do not match the distances between the teethes of the sprocket roller, it causes movie tape displacements each time the transportation teeth enters the perforation hole. This is presented in fig. 2. Different reasons for wow and flutter can be found in magnetic and mechanical recordings [JAES]. Again most of them are triggered by the poor equipment or lack of appropriate production standards, thus are highly susceptible in the ethnic recordings. The authors of this papers presented several DSP algorithms for correcting wow and flutter. The algorithms utilize some ancillary information retrievable from the audio documents, i.e. the initial frequencies of the power line hum, of the high frequency bias or of the 15.734 kHz pilot tone, which can be found the NTSC television audio recordings. Short overview of those will be presented in the next section whereas the details can be found in the literature [JAES], [Pastuszak]. New approach was recently studied by the authors [AESCon]. It is dedicated to movie soundtracks and utilizes image processing of the movie tape where the ancillary historical information like the tape format, its initial speed or the recording procedure play the crucial role. After providing those information, which can be very unique for some of the obsolete recordings, the reduction of the pitch variations of the movie soundtrack can be greatly improved and even done automatically for some cases.

2. State of the Art

The idea of DSP algorithms for wow and flutter determination and reduction was introduced by Gerzon [Gerzon]. He suggested using the magnetic leftover of the high-frequency bias, which can be found in magnetic tape recordings, as the pilot tone depicting the distortions’ course. Additionally, he proposed the nonuniform resampling technique for reducing the distortions. However, Gerzon did not verified his ideas.

First DSP algorithms for wow course determination and reduction were presented by Godsill [Godsill 1]. Then, further developed, they were reported by him together with several co-researchers [Godsill 2], [Godsill 3], [Godsill 4], [Godsill 5] . The algorithms aimed at the automatic correction of smooth pitch variations over long time scales. They were build on the three-step processing scheme. First the preprocessing, i.e. the standard sinusoidal modelling analysis, provided information on the frequency partials. Secondly the partials were analyzed in order to determine the wow course. Thirdly the nonuniform resampling was used for the distortion reduction. The main emphasis was put on the second step, i.e. the determination of the wow course. Here different a priori models of the distortion course, ranging from simple sinusoidal to more complex regressive processes, were examined using the Bayes’ estimation scheme. It was thru those models that the ancillary information about the recording medium, gramophone records in this case, could be used.

Partials’ processing was also used by Nichols in his approach to the wow determination and reduction [Nichols 1], [Nichols 2]. The first among his algorithms was similar to the Gosill’s approach with only minor improvements added, such as the spectrum warping or the iterative procedure for improving the spectrum’s peaks frequency estimation. Second algorithm was build on the novel concept of graphical processing of the spectrogram. In this algorithm partials were build using image processing routines which allowed to select the pixels representing the tonal components in the spectrogram. In both of Nichol’s algorithms, partials were identically processed in order to determine the wow course. As the algorithms designed by Nichols were dedicated to phonograph recordings restoration, the simple sinusoidal model was used to describe wow. The non-uniform resampling was then applied to reduce the distortions.

Howart and Wolfe reported algorithms for wow and flutter determination and removal [H&W]. They designed a system being a combination of a hardware part for capturing the high frequency bias and a software part for wow and flutter reduction. Additionally, those authors presented also the study of the non-uniform resampling technique used for wow and flutter reduction [W&H].

Wow and flutter determination and reduction was studied also by the authors of this paper. Different algorithms were proposed for wow course determination such as [JAES]: the power line hum tracker, the high frequency bias tracker, which was also successfully used for the multichannel television sound pilot tone tracking  [Pastuszak], as well as the spectrum centre of gravity analyser. The algorithms were tested using real-life archive recordings [JAES]. Simulation were also made in order to determined their limitation in terms of the maximal modulation frequency [Maziewski 1]. It was found that only the high frequency bias tracker can determined both wow and flutter courses, whereas the other algorithms can process flutter only to some extent. We have also studied the non-uniform resampling influence on the quality of the restored recordings [Maziewski 2].

All of the algorithms for wow and flutter correction presented above were based on a similar idea. The audio documents’ content, either the audio itself or additional signals like the power line hum or the high frequency bias, were analyzed in order to determine the distortion course. Meanwhile a different approach can be applied: the analysis of the audio carrier. Some interesting work related to his subject already exists. I.e. Fedeyev and Haber proposed image processing of the mechanically made recordings (modulated groves) [F&H]. In their approach the grove shapes are being scanned, their images corrected, and then translated into sound. Image corrections allow to virtually repair even gross carrier damages. Such processing allows also for wow and flutter reduction but only to the ones triggered by the ellipticity or eccentricity of the black record. Sounds from other carriers can be similarly restore. It was reported that image processing can be used for movie soundtrack restoration [B&H&B&D], [R&P&K].

Image processing of the optical tapes can be used for wow and flutter determination and reduction in movie soundtracks. The algorithms presented in the next section allows to determined the course of wow and to some extend the course of flutter.

3. New Approach

The idea behind the algorithms is to use image processing for calculating the local tape shrinkage which is one of the reasons for drift, wow and flutter. The shrinkage can be calculated via analyzing the image height of: a movie frame, sprocket hole, pitch or another standardized movie tape element; and then it can be expressed as the drift, wow and flutter characteristic. After the characteristic determination both the soundtrack and movie frames can be corrected.

This methods can be used to reconstruct soundtracks from 35mm and 16mm film with optical soundtracks. In films where sound was recorded at magnetic soundtrack only image of movie frame can be corrected. It is expected that movie frames will be digitalize properly with high spatial resolution and at least  48 bits colour depth. Properly scanned images means that the image of movie frame is parallel to  edge of scanned image and sharpness of picture is correct.

One PVC value can be calculated for each movie frame. This value is defined as the local tape shrinkage which affects this particular frame. For the first frame, it can be expressed as:

PVC(t1)=Fheigth1Fheigthnom	(1)

where the Fheigthnom represents the nominal frame height (for a standard tape without any shrinkage) and. the time point t1 can be expressed in seconds as:

t1=Fheigth12H1sec	(2)

where the H1sec represents the movie tape height of a one second film  recording.

Successive PVC values can be calculated as:

PVC(tn)=FheigthnFheigthnom	(3)

Similar to eq. 2the time point tn can be expressed in seconds as:

tn=m=1n−1Fheigthm+Fheigthn2H1sec	(4)

In the next section the algorithm for frames (or perforation) height calculation is presented.

3.1 Frame height calculation

To calculate frame height we have to know how movie frames looks. There are a little differences between frames in 35mm and 16mm films. In 35mm films there is black area between two frames as it is shown in figure XXX1 and frame in 16mm films are located close to each other without any black area. Although the same image processing can be used for 35mm and 16mm films.

To determine size of movie frame there is simple edge detection algorithm used. It is possible to used this  algorithm because the shape of elements that are measured is regular and not complicated . So only location of two parallel lines corresponding to frame beginning and end located perpendicular to  soundtrack image have to be find.

In this method of PVC determination  the image of the movie frame contains whole current processed frame and a part of frame before and behind it. It is necessary to have data redundancy to see where image of current movie frame begins and ends. To calculate frame height image of film is cut and decimal-to-binary conversion is made. This operation haul up edges. Then mean value of every row of image pixels is calculated. Analyzing  values of the mean signal helps to determine beginning and the end of two contrast areas. To do this more accurately from the mean signal  derivate is calculated and instead of mean signal analyzing, derivative of it is processed. As it is shown at the figure XXX2 beginning and the end of the frame corresponds to local maximal values of the derivate signal.

3.1 Algorithm for calculating height of tape perforation

This method of PVC determination give improvement of quality only for 35mm films, because at this type of films one move frame  corresponds to four perforation holes and at 16mm films one movie frame corresponds only to one perforation hole . So usage of this algorithm for  35mm film allows to determine four times higher modulation frequency than in algorithm that determine PVC from movie frame height. Way of  finding beginning and end of  perforation holes is similar to height of movie frames calculation. In this algorithm images of film tape contains only image of current processed movie frame and image of perforation holes that corresponds to this frame.

From whole movie frame image perforation image is cut and perforation size is calculated from it. To determine size of holes decimal-to-binary conversion is made like in frame height calculation algorithm. Next mean value of image rows is calculated and from this signal derivate is taken. Beginning of the first perforation hole corresponds to the image beginning and beginning of the last  perforation holes corresponds to the image end. Other beginnings of  the perforation are calculated as local maxims of derivate from mean value signal. Height of perforation hole that is analyze in this PVC determination algorithm isn’t really a height of it but a distance between two beginnings of neighbouring perforation holes.

4. Experiments / comparison of the new method against known approaches

The experiments aim at the comparison of the new method for wow and flutter determination and reduction with the known approaches. Two factors are taken into consideration. The first one is the maximal modulation frequency (fmax) which can be captured in the determined PVC. The second one is the precision of the determined PVC.

The preliminary study of the fmax limitation for the new method was done by the authors of this paper [AESCon]. It was shown that the limitation depends on the film format and the movie tape velocity, which can be expressed using either in frame per second (ffp) or perforation per second (pps) characteristics. The determined values of the fmax are given in table 1 and 2.

Table 1 Frame-based fmax limitations [AESCon]

| Film format [mm] | 8 | 16 | 35 | 65/70 |
| --- | --- | --- | --- | --- |
| Tape velocity [fps] | 16 | 24 | 24 | 24 |
| fmax [Hz] | <3.2 | <4.8 | <4.8 | <4.8 |

Table 2 Perforation-based fmax limitations [AESCon]

| Film format [mm] | 8 | 16 | 35 | 65/70 |
| --- | --- | --- | --- | --- |
| Tape velocity [pps] | 16 | 24 | 96 | 120 |
| fmax [Hz] | <3.2 | <4.8 | <19.2 | <72 |

As the 35 mm film is one of the most popular formats in film archives [2JAES] it was chosen for the experiment.  Additionally, as the perforation-based fmax is higher for this film format, the perforation-based processing was used in the performed simulations.

Among the reported audio-based algorithms used for the PVC determination (see Section 2) only tree have been studied in terms of the fmax limitation [Maziewski 1]. Those are: the power line hum tracker, the centre of gravity (COG) tracker and the high frequency bias tracker. The results of this study are given in table 3. In the table the Fs values stand for the sampling frequency of the digitized audio recordings, whereas M represents the processing frame length.

Table 3. fmax limitations for the wow and flutter tracking algorithms using audio analysis [Maziewski 1]

| Algorithms and their settings | fmax |
| --- | --- |
| Power line hum tracker; M = 8 Sa; Fs =200 Hz; | < 12.5 Hz |
| COG tracker; M = 1024 Sa; Fs =44100 Hz; | < 21.5 Hz |
| High-frequency bias tracker; M = 1024 Sa; Fs =192000 Hz | < 93.75 Hz |

Among those tree algorithms the power line hum tracker is the most appropriate for the comparison with the new method. It is because it can be used almost automatically, avoiding problems with the wow versus vibrato comparison. Additionally, power line hum can be easily found in many of the film soundtracks (which is true for the recordings from the Polish film archives).

The experiments were performed as follows. First the wow and flutter simulations were prepared. For the power line hum tracker 4 sec long sound files with 50 Hz and 60 Hz tones were prepared, simulating the European- and USA-originated recordings respectively. For the new method 100 movie frames (digital images) in the 35 mm format were prepared, which corresponds to a 4 sec long audio and video recording, assuming the 25 fps tape velocity. As the typical tape resolution lies between 60 to 80 lines per mm [Malecki], which represents the range from 1524 DPI to 2032 DPI, the 3600 DPI image resolution was chosen to satisfied the most demanding cases.

Both the sound file and the movie frames were frequency modulated using a complex characteristic with the limited maximal frequency. The characteristic was generated as the white Gauss noise low-pass filtered with the cut-off frequency equal to the fmax limitation. Five fmax values were studied: 0.5 Hz, 6 Hz, 12 Hz, 19 Hz and 38 Hz. First two corresponded to the wow definition (form 0.5 Hz to 6 Hz). Third one was the fmax limitation of the power line hum tracker (see table 3). The fourth was the fmax limitation of the new method (see table 2) whereas the last value doubled that limitation. Frequency modulation was performed using the non-uniform resampling: one dimensional for the sound files and two-dimensional for the movie frames. Two values of the maximal allowable relative deviation of the modulation characteristics (i.e. modulation depth) were used in the experiment: 1.1 and 1.01. The first value was used for simulating gross pitch variations whereas the second value for simulating  smaller ones. The simulations were repeater 100 times for each algorithm and for all fmax and maximum deviation values. The mean square error (MSE) between the simulated and determined PVCs was calculated for each simulation. Then the results were summarized using the mean and standard deviation values for all of the calculated MSEs.

Results obtained using the power line hum tracker are given in tables 4 to 7. First the results for the simulations involving the 50 Hz tone  are given. Then they are followed by the results for the sound files with the 60 Hz tone. In each case the results from the simulations with the gross deviations are presented first.

Table 4. MSE of the PVCs determined using the power line tracker, 50 Hz tone, maximal allowable deviation 1.1

| fmax [Hz] | 0.5 | 6 | 12 | 19 | 38 |
| --- | --- | --- | --- | --- | --- |
| Mean Value (Std. Deviation) | 1,48E-05 (1,10E-05) | 8,38E-05 (2,78E-05) | 2,04E-04 (4,16E-05) | 3,63E-04 (6,99E-05) | 5,03E-04 (9,40E-05) |

Table 5. MSE of the PVCs determined using the power line tracker, 50 Hz tone, maximal allowable deviation 1.01

| fmax [Hz] | 0.5 | 6 | 12 | 19 | 38 |
| --- | --- | --- | --- | --- | --- |
| Mean Value (Std. Deviation) | 1,29E-05 (8,78E-06) | 1,36E-05 (8,05E-06) | 1,69E-05 (7,06E-06) | 1,96E-05 (7,10E-06) | 1,91E-05 (6,70E-06) |

Table 6. MSE of the PVCs determined using the power line tracker, 60 Hz tone, maximal allowable deviation 1.1

| fmax [Hz] | 0.5 | 6 | 12 | 19 | 38 |
| --- | --- | --- | --- | --- | --- |
| Mean Value (Std. Deviation) | 1,69E-05 (1,19E-05) | 1,31E-04 (7,18E-05) | 2,61E-04 (5,14E-05) | 4,00E-04 (7,27E-05) | 5,27E-04 (8,20E-05) |

Table 7. MSE of the PVCs determined using the power line tracker, 60 Hz tone, maximal allowable deviation 1.01

| fmax [Hz] | 0.5 | 6 | 12 | 19 | 38 |
| --- | --- | --- | --- | --- | --- |
| Mean Value (Std. Deviation) | 8,27E-06 (6,23E-06) | 8,69E-06 (5,64E-06) | 1,18E-05 (5,91E-06) | 1,24E-05 (5,49E-06) | 1,38E-05 (5,11E-06) |

The results obtained for the new method are given in tables 8 and 9.

Table 8. MSE of the PVCs determined using the local tape shrinkage analysis, maximal allowable deviation 1.1

| fmax [Hz] | 0.5 | 6 | 12 | 19 | 38 |
| --- | --- | --- | --- | --- | --- |
| Mean Value (Std. Deviation) | 5,09E-05 (4,74E-05) | 5,02E-05 (5,43E-05) | 8,09E-05 (0,000154) | 9,05E-05 (0,000153) | 1,01E-04 (0,000179) |

Table 9. MSE of the PVCs determined using the local tape shrinkage analysis, maximal allowable deviation 1.01

| fmax [Hz] | 0.5 | 6 | 12 | 19 | 38 |
| --- | --- | --- | --- | --- | --- |
| Mean Value (Std. Deviation) | 2,32E-05 (2,99E-07) | 2,32E-05 (2,63E-07) | 2,32E-05 (2,84E-07) | 2,32E-05 (2,22E-07) | 2,32E-05 (2,92E-07) |

To visualise the obtained results better additional charts are given. Fig. 7 and 8 present the box-plots of the mean MSEs obtained for the high and low deviations respectively.

For the simulations involving samples with the higher relative deviations it can be seen that the new method provides better results. The overall MSE, calculated for all of the examined fmax values, is smaller for this method comparing to the total MSE of the power line hum tracker. For both algorithms the MSE increases with the higher fmax values, however, the degree of increase is smaller for the new method.

For the simulations involving samples with the smaller relative deviations the situation is different. The overall MSE is higher for the new method. It can be seen that the mean errors are the same for all of the examined fmax values (here only the standard deviation values allows to differentiate results). The high and constant values of the mean MSEs are caused by the precision limitation of the proposed method. Using the 3600 DPI resolution images the distance between successive perforation holes equals 670 pixels. Knowing that the maximal allowable deviation is only 1.01, the maximal length in the modulated perforations’ distances equals 7 pixels, and in most cases is less than that. Comparing this with the limited precision of  perforation distance evaluation of the new method, which is +/- 2 pixels, explains the reported constant value of the MSE.

5. Conclusions

Comparison:

Will conclude the modulation frequency range comparison as well as the other advantages and disadvantages of the new method compared against the known approaches.

6. Summary

Acknowledgements

This work was supported by the Polish Government research resources for years 2007-2009 as the research project no. 4381/B/T02/2007/33.

Appendix

References

[AES stand] AES Std. 6-1982, Method for Measurement of Weighted Peak Flutter of Sound Recording and Reproducing Equipment, Audio Engineering Society, New York, 2003.

[1JAES] E. D. Daniel, C. D. Mee, M. H. Clark, Magnetic Recording: The First 100 Years, IEEE Press, New York, 1999.

[2JAES] P. Read and M. P. Meyer, Restoration of Motion Picture Film, Butterworth Heinemann, Oxford, UK, 2000.

[3JAES] E. S. Busby, Analog Tape Recording, in: J. Whitaker, B. K. Benson (Eds.), Standard Handbook of Audio and Radio Engineering, McGraw-Hill, New York, 2003, pp. 6–21-6–42.

[FN] Pogrzeb Marszałka Piłsudskiego, 35 mm film from the Polish Film Archives, Warsaw 1961 copy.

[Malecki] I. Malecki, Technika Nagrywania i Odtwarzania Dzwiekow, PWT, Warszawa 1953. (in Polish)

[JAES] A. Czyzewski, A. Ciarkowski, A. Kaczmarek, J. Kotus, M. Kulesza, P. Maziewski, DSP Techniques for Determining “Wow” Distortion, Journal of the Audio Engineering Society 55 (4) (2007) 266-284.

[Pastuszak] P. Pastuszak, MTS Pilote Tone Tracking for “Wow” Determination, Archives of Acoustics 33 (1) (2008) 117-123.

[AESCon] P. Maziewski, A. Kupryjanow, A. Czyżewski, Drift, Wow and Flutter Measurement and Reduction in Shrunken Movie Soundtracks, presented at the 124th AES Convention, Amsterdam, The Netherlands, 17-20 May 2008, preprint no. 7392.

[Gerzon] M. Gerzon, Don’t Destroy the Archives!, unpublished (1992); available at ww.waves.com/german/htmls/service/faq/dont.html (accessed 2006 Oct.).

[Godsill 1] S. J. Godsill, The Restoration of Degraded Audio Signals, Ph.D. dissertation, Cambridge University, Cambridge, UK, 1993.

[Godsill 2] S. J. Godsill, P. Rayner, The Restoration of Pitch Variation Defects in Gramophone Recordings, Proc. IEEE Workshop on Applications of Signal Processing to Audio and Acoustics, Mohonk, NY, 17-20 October 1993, pp. 148-151.

[Godsill 3] S. J. Godsill, Recursive Restoration of Pitch Variation Defects in Musical Recordings, Proc. IEEE Int. Conf. on Acoustics, Speech and Signal Processing 2, Adelaide, Australia, 19-22 April 1994, pp. 233-236.

[Godsill 4] S. J. Godsill, P. Rayner, Digital Audio Restoration - A Statistical Model-Based Approach, Springer, London, 1998, Chapter 8, pp. 171–190.

[Godsill 5] S. J. Godsill, G. Reid, and C. Hicks, “Restoration of Smooth Pitch Variations over Long Timescales,”  presented at the Joint Technical Symposium, Toronto, Canada, 24–26 June 2004; available at http://www.jts2004.org/english/proceedings/Reid.htm (accessed October 2008).

[Nichols 1] J. Nichols, An Interactive Pitch Defect Correction System for Archival Audio, Proc. AES 20th Int. Conf., Budapest, Hungary, 5-7 October 2001, paper no. 1941.

[Nichols 2] J. Nichols, High Performance, Low-Cost Wax Cylinder Transcription System, Proc. AES 20th Int. Conf., Budapest, Hungary, 5-7 October 2001, paper no. 1937.

[H&W] J. Howarth, P. Wolfe, Correction of Wow and Flutter Effects in Analog Tape Transfers, presented at the 117th AES Convention, San Francisco, USA, 28-31 October 2004, preprint no.6213.

[W&H] P. Wolfe, J. Howarth, Nonuniform Sampling Theory in Audio Signal Processing, presented at the 116th AES Convention, Berlin, Germany, 8-11 May 2004, preprint no. 6123.

[Maziewski 1] P. Maziewski, Modulation Frequency Constrains on Wow and Flutter Determination, Archives of Acoustics 33 (1) (2008) 125-131.

[Maziewski 2] P. Maziewski, Wow defect reduction based on interpolation techniques, Bulletin of the Polish Academy of Science: Technical Sciences 54 (4) (2006) 469-477; available at http://bulletin.pan.pl/(54-4)469.pdf (accessed October 2008).

[F&H] V. Fadeyev, C. Haber, A novel application of high energy physics technology to the problem of audio preservation, Nuclear Instruments and Methods in Physics Research A 518 (2004) 456-462.

[B&H&B&D] E. Brun, A. Hassaine, B. Besserer, E. Decenciere1, Restoration of Variable Area Soundtracks, IEEE International Conference on Image Processing, ICIP 2007, San Antonio, Texas, USA, 16-19 September 2007.

[R&P&K] D. Richter, D. Poetsch, A, Kuiper, Localization of Faults in Multiple Double Sided Variable Area Code Sound Tracks on Motion Picture Films Using Digital Image Processing, 13th International Czech-Slovak Scientific Conference Radioelektronika, 2003.

Vitae

Figures

Fig. 1 Example of badly produced movie tape splice [FN]

Fig. 2 Tape shrinkage effects on the movie tape transportation system [Malecki]

Fig. XXX1 Example of 35mm and 16mm films

Fig. 7 Mean MSEs obtained for the simulations with the high relative deviations.

Fig. 8 Mean MSEs obtained for the simulations with the small deviation.

Legends

Tables

Outline:

- Introduction (PM) 1str

Short introductory part indicating the purpose of this work, presenting basic definitions (i.e. parasitic pitch variations – mainly wow and flutter), examples of deformed movie tapes, mechanisms responsible for those distortions, known analogue approaches to the reduction of those distortions, etc.

- State of the Art (PM) 2 str

Overview of the DSP-based methods for wow and flutter reduction (reference-based, i.e. JAES). It should be indicated here that some of the methods already utilize “the ancillary information, retrievable from the history of the audio document and from the history of the musical technology connected with it” (quotation from the “aim of the special issue”). Those are the “pilot tone analysis” methods. However, those are not always applicable, i.e. special features are needed like the high frequency bias. On the other hand methods which rely only on audio analysis can produce false results, e.g. wow vs. vibrato discrimination. Incorrect results are highly susceptible when analysing unique ethnic audio documents which have no reference scores or similar performances to camper with. In such situation the new approach i.e. audio carrier analysis, allows to overcome some of the reported problems.

- New Approach (AK) 2 str

The idea behind the algorithms is to use image processing for calculating the local tape shrinkage which is one of the reasons for drift, wow and flutter. The shrinkage can be calculated via analyzing the image height of: a movie frame, sprocket hole, pitch or another standardized movie tape element; and then it can be expressed as the drift, wow and flutter characteristic. After the characteristic determination both the soundtrack and movie frames can be corrected.

- Experiments (PM, AK) 3 str

Experiments will cover modulation frequency range estimation for the new and the old algorithms. Estimated values will allow to compare the algorithms.

- Conclusion 0,5 str

Will conclude the modulation frequency range comparison as well as the other advantages and disadvantages of the new method compared against the known approaches.

- Summary 0,5 str

- References 1 str

Form: "Fast Communication", 10 pp, single column, double spaced.

Format:

- The manuscript must be written in English and the text of the paper should be preceded by an abstract of no more than 200 words.

- The first page should include the article title and the author's name and affiliation, as well as a name and mailing address to be used for correspondence and transmission of proofs.

- The second page should include a list of unusual symbols used in the article and the number of pages, tables and figures. It should also contain the keywords in English.

- A Fast Communication is a short, self-contained article not exceeding 10 pages (single column, double spaced) in length on ongoing research, or reporting interesting possibly tentative ideas, or comments on previously published research. The editorial decision is typically binary to provide rapid dissemination of the results. The objective is to provide detailed, constructive feedback on submitted papers and publish high quality papers within a very short period of time. The target for a first reply is two months.

- "Research article" (up to 30 pp). Research articles should not exceed 30 pages (single column, double spaced) in length and must contain novel research within the scope of the journal.

- The text should be in single-column format

- do not use the wordprocessor's options to justify text or to hyphenate words

- Do not embed "graphically designed" equations or tables, but prepare these using the wordprocessor's facility

- When preparing tables, if you are using a table grid, use only one grid for each individual table and not a grid for each row. If no grid is used, use tabs, not spaces, to align columns. The electronic text should be prepared in a way very similar to that of conventional manuscripts

- Do not import the figures into the text file but, instead, indicate their approximate locations directly in the electronic text and on the manuscript

- References:

  - Reference to a book:
[1] M. Kunt, Digital Signal Processing, Artech House, Norwood, MA, 1986, Chapter 6, pp. 319-338.

  - Reference to a journal publication:
[2] F.J. Harris, On the use of windows for harmonic analysis with the discrete Fourier transform, Proc. IEEE 66 (1) (1978) 53-83.

  - Reference to a conference paper:
[3] D. Coulon and D. Kayser, A supervised-learning technique to identify short natural language sentence, Proc. 3rd Internat. Joint Conf. on Pattern Recognition, Coronado, CA, 8-11 November 1976, pp. 85-89.

  - Reference to a chapter in an edited book:
[4] E.F. Moore, The firing squad synchronization problem, in: E.F. Moore (Ed.), Sequential Machines, Selected Papers, Addison-Wesley, Reading, MA, 1964, pp. 213-214.