Title: Reduction of Parasitic Pitch Variations in Ethnic Music.

Authors: Andrzej Czyzewski, Przemyslaw Maziewski, Adam Kupryjanow

Affiliation: Gdansk University of Technology, Multimedia Systems Department, ul. Narutowicza 11/12, 80-226 Gdansk, POLAND; {andcz, przemas, adamq}@sound.eti.pg.gda.pl, tel. +48 58 3471301, fax +48 58 3471114.

Abstract: This paper presents the new method for reducing the parasitic pitch variations in archival audio recordings. The new method is intended for analyzing the movie soundtracks recorded in optical films. It utilizes image processing for calculating and reducing the effects of tape shrinkage which is one of the main reasons for parasitic pitch variation in film audio. As long as the film tape characteristics are known the new method can be easily tuned to analyzed obsolete, non-professional archival recordings. In the paper the new method is compared with the previous approaches.

List of unusual symbols used in the article:

fm - parasitic modulation frequency

PVC(t) - pitch variation curve

Pdistnom - standardized distance between two perforation holes

Pdistn - real distance between the n-th and the n+1perforation hole

H1sec - length of a one second long movie tape recording

tn - n-th time position of the calculated PVC

fmax - maximal modulation frequency in the determined pitch variation curve

Fs - sampling frequency of a digitized audio recording

M - processing frame length

Number of:

pages - 20, tables - 9, figures - 9.

Keywords: audio restoration, parasitic pitch variations, wow and flutter.

1. Introduction

The purpose of this work is to present a new method for reducing parasitic pitch variations in archival audio recordings and compare it with the known approaches. Parasitic pitch variations, mainly wow and flutter, can be defined as unwanted frequency modulations added to the original signal during recording, producing, monitoring or duplicating phases. Wow and flutter are defined based on the range of the parasitic modulation frequency (fm): wow fm ∈〈0.5 Hz, 6 Hz), flutter  fm ∈〈6 Hz, 100 Hz〉 [1]. The distortions are introduced to the signal by the irregular velocity of its analogue carrier. The irregularities can originate from various mechanisms, depending on the carrier type, its condition and damages, its production technique, and other factors [2], [3], [4]. The irregularities are commonly found in the ethnic music recordings made by an unskilled staff using nonprofessional recording systems. Especially in the eastern Europe low quality of the audio carriers, poor alignment and maintenance of the recording and reproducing equipment enhanced the risk of wow and flutter.

Example of a badly produced movie tape splice, taken from a Polish archive film, is given in fig. 1.

Fig. 1 Example of a badly produced movie tape splice

When such a tape makes a contact with the projector’s sprocket roller (or telecine’s) it moves slightly back or towards its original movement direction, depending on the difference between the interval of successive perforation holes and the distance between the teeth of the transpiration system (see fig. 2). Both cases result in a momentary pitch variation of the soundtrack.

The other reason for wow and flutter in the ethnic movie soundtracks is the tape shrinkage. It is caused by loss of water, solvent and plasticizer in movie tapes. Both nitro based and acetate based films are exposed to this process. Movie tape shrinkage causes similar problems as the ones presented with the badly produced splice. As the shrink perforation do not match the distances between the teethes of the sprocket roller, it causes movie tape displacements each time the transportation teeth enters the perforation hole. This is presented in fig. 2.

Fig. 2 Tape shrinkage effects on the movie tape transportation system [5]

Different reasons for wow and flutter can be found in magnetic and mechanical recordings [6]. Again most of them are triggered by the poor equipment or lack of appropriate production standards, thus are highly susceptible in the ethnic recordings. The authors of this papers presented several DSP algorithms for correcting wow and flutter. The algorithms utilize some ancillary information retrievable from the audio documents, i.e. the initial frequencies of the power line hum, of the high frequency bias or of the 15.734 kHz pilot tone, which can be found in the NTSC television audio recordings. Short overview of those will be presented in the next section whereas the details can be found in the literature [6], [7]. New approach was recently studied by the authors [8]. It is dedicated to movie soundtracks and it utilizes image processing of the movie tape where the ancillary historical information, like the tape format, its initial speed or the recording procedure, play the crucial role. After providing those information, which can be very unique for some of the obsolete recordings, the reduction of the pitch variations of the movie soundtrack can be greatly improved and even done automatically. This new approach will be presented in details in this paper and then compared with the previously elaborated algorithms.

2. State of the Art

The idea of DSP algorithms for wow and flutter determination and reduction was introduced by Gerzon [9]. He suggested using the magnetic leftover of the high-frequency bias, which can be found in magnetic tape recordings, as the pilot tone depicting the distortions’ course. Additionally, he proposed the non-uniform resampling technique for reducing the distortions. However, Gerzon did not verified his ideas.

First DSP algorithms for wow course determination and reduction were presented by Godsill [10]. Then, further developed, they were reported by him together with several co-researchers [11], [12], [13], [14] . The algorithms aimed at the automatic correction of smooth pitch variations over long time scales. They were build using the three-step processing scheme. First the preprocessing, i.e. the standard sinusoidal modelling analysis, provided information on the frequency partials. Secondly the partials were analyzed in order to determine the wow course. Thirdly the non-uniform resampling was used for the distortion reduction. The main emphasis was put on the second step, i.e. the determination of the wow course. Here different a priori models of the distortion course, ranging from simple sinusoidal to more complex regressive processes, were examined using the Bayes’ estimation scheme. It was thru those models that the ancillary information about the recording medium, gramophone records in this case, could be used.

Partials’ processing was also used by Nichols in his approach to the wow determination and reduction [15], [16]. The first among his algorithms was similar to the Gosill’s approach with only minor improvements added, such as the spectrum warping or the iterative procedure for improving the spectrum’s peaks frequency estimation. Second algorithm was build on a novel concept of graphical processing of the spectrogram. In this algorithm partials were build using image processing routines which allowed to select the pixels representing the tonal components from the spectrogram. In both of Nichol’s algorithms, partials were identically processed in order to determine the wow course. As the algorithms designed by Nichols were dedicated to phonograph recordings restoration, the simple sinusoidal model was used to describe wow. The non-uniform resampling was then applied to reduce the distortions.

Howart and Wolfe reported algorithms for wow and flutter determination and removal [17]. They designed a system being a combination of a hardware part for capturing the high frequency bias and a software part for wow and flutter reduction. Additionally, those authors presented also the study of the non-uniform resampling technique used for wow and flutter reduction [18].

Wow and flutter determination and reduction was studied also by the authors of this paper. Different algorithms were proposed for wow course determination such as [6]: the power line hum tracker, the high frequency bias tracker, which was also successfully used for the multichannel television sound pilot tone tracking  [7], as well as the spectrum centre of gravity analyser. The algorithms were tested using real-life archive recordings [6]. Simulation were also made in order to determined their limitations in terms of the maximal modulation frequency [19]. It was found that only the high frequency bias tracker can determined both wow and flutter courses, whereas the other algorithms can process flutter only to some extent. We have also studied the non-uniform resampling influence on the quality of the restored recordings [20].

All of the algorithms for wow and flutter correction presented above were based on a similar idea. The audio documents’ content, either the audio itself or embedded signals like the power line hum or the high frequency bias, were analyzed in order to determine the distortion course. Meanwhile a different approach can be applied: the analysis of the audio carrier. Some interesting work related to his subject already exists. I.e. Fedeyev and Haber proposed image processing of the mechanically made recordings (modulated groves) [21]. In their approach the grove shapes are being scanned, their images corrected, and then translated into sound. Image corrections allow to virtually repair even gross damages of the carrier. Such processing allows also for wow and flutter reduction but only to the ones triggered by the ellipticity or eccentricity of the black records. Sounds from other carriers can be similarly restore. It was reported that image processing can be used for movie soundtrack restoration [22], [23].

Image processing of the optical tapes can be used for wow and flutter determination and reduction in movie soundtracks. The algorithms presented in the next section allows to determined the course of wow and to some extend the course of flutter and then to reduce it.

3. New Approach

Typically in the DSP algorithms for wow and flutter determination this distortions are characterized using the pitch variation curve (PVC) [6]. This curve depicts the distortions course showing both: their depth and frequency. The PVC can be estimated using the tape shrinkage analysis as shrinkage results in pitch changes of the soundtrack – assuming constant speed of the carrier. To estimate the local tape shrinkage, heights of selected elements of the movie tape must be compared against their original values. The original values must be know a priori. In most cases this knowledge comes from the standardization (16 mm or 35 mm). In rare ethnic recordings, made with obsolete formats, the knowledge about the movie tape format must be obtained from the historical sources or presumptions must be made and then corrected after obtaining the first results.

For the presented method to work it is expected that the movie tape will be digitalize properly with a high spatial resolution and at least 48 bit colour depth. Proper scan is obtained when the image of the movie frame is parallel to the edge of the scanned image and the sharpness of picture is correct. In the fig. 3 image of a properly scanned, typical movie frames is shown.

Fig. 3 Example of a typical 35 mm movie frame

The PVC can be estimated via analysis of the standardized elements of the movie tape (i.e. distances between frames or perforation holes). When analysing the perforation of the film the first value of the PVC is determined in accordance with expression (1)

PVC(t1)=Pdist1Pdistnom	(1)

where Pdistnom represents the known (standardized) distance between two perforation holes, Pdist1 is the distance between the first and the second perforation hole in the analyzed movie (see fig. 3) and t1 is the time position of the calculated PVC value and it can be expressed in seconds as

t1=Pdist12H1sec	(2)

In eq. 2 the H1sec represents the length of a one second long movie tape recording.

The successive PVC values can be calculated as

PVC(tn)=PdistnPdistnom	(3)

where the tn represents the time position of the n-th PVC value and it can be calculated in seconds as:

tn=m=1n−1Pdistm+Pdistn2H1sec	(4)

In the next section algorithms for calculating the distances between successive perforation holes as well as between the frame beginnings will be presented.

3.1 Algorithm for calculating the distances between successive perforation holes

As the perforation holes have rectangular shape (or are close to it) the Pdistn can be calculated using the locations of two parallel lines representing the successive perforation beginnings. In the presented algorithm the edge detection is applied to determine the perforation beginnings. The image colour is analysed for capturing the transition between the darkest colour – representing the perforation hole border – and the lighter colours representing either the tape or the hole. Scheme of this algorithm is presented in fig. 4.

Fig. 4 Algorithm for calculating the distances between successive perforation holes in one movie frame

In the algorithm only one frame perforation image is being analyzed. The processing is repeated for all of the successive frames. The first processing block represents the cropping procedure where the image is cut accordingly to the perforation width. To facilitate the determination of the perforation distances the decimal-to-binary conversion is made, emphasising perforation borders. This results in a black and white image. Further the processed image is analyzed as a two dimensional matrix. First the mean value in each column is calculated and then its approximate derivate (i.e. differences). The obtained vector corresponds to the intensity changes in the perforation’s picture. Its maximal values represents the beginning of the successive perforation holes (see fig. 5). The Pdistn is calculated as the difference of the two successive perforation holes beginnings.

Fig. 5 Example of determined distances between successive perforation holes for the 35 mm film (one frame)

Analysing the perforation in the 35 mm films allows to determine four PVC values for each movie frame. It is different however with the 16 mm films where only one perforation hole comes with each movie frame. In such cases, or when the perforation is not available because it was not scanned, the movie frame analysis can applied for the Pdistn calculation.

3.2 Algorithm for calculating the distances between successive movie frames

Movie frames heights can be calculated using the similar processing scheme as the one presented in fig. 4. It is because their share the same characteristics as the perforations, i.e. they are rectangular with well defined borders. In case of movie frames in the first processing step the perforation must be cut out from the analyzed image. Then the same processing is applied as for the perforation. The maxima of the obtained intensity changes vector can now be used to determined the movie frames beginnings (see fig. 6).

Fig. 6 Example of determined distances between successive movie frames for the 35 mm film

Knowing the PVC allows to reduce the soundtrack’s wow and flutter caused by the tape shrinkage. Reduction can be done either via the soundtrack picture processing - inverting the shrinkage - or it can done for the digital sound obtained from the soundtrack. The first approach involves the two-dimensional and the second one the one-dimensional non-uniform resampling [20]. In both cases the algorithm for translating the image of the soundtrack to the digital sound is useful.

3.3. Translating the image of the soundtrack into the digital sound

The translation algorithm was designed to process movie soundtrack frame-by-frame. In order to concatenate the sound buffers from successive frames a redundancy must be provided in the processed images. The image of one movie frame must contain part of the previous and the next frame (like in fig. 3) . The scheme of translation algorithm is presented in fig. 7.

Fig. 7 Scheme of algorithm for translating the image of the optical soundtrack to the digital sound

In the preprocessing stage the image of the soundtrack is edit out from entire frame. Then the algorithm follows the processing scheme of the standard cinema film projector. In the projector the amplitude of the sound signal is determine as the intensity of the light that has passed thru the soundtrack. Thus to calculate the intensity of light the mean value in columns of the soundtrack image are calculated. The obtained vector of mean values represents the amplitudes of the sound signal. As in most cases the obtained signal has gross constant component its mean value is subtracted.

If more than one image frame has to be translated, sound buffers from each successive frame have to be connected. Concatenation is made by calculating the cross-correlation function between the sound signal taken from the end of the image of the previous frame and the sound signal taken from the beginning of the image of the currently processed frame. The maximum value of the cross-correlation function determines the similarity region of the sound signals from the previous and the current buffer, which allows to connected and mixed them both.

When the digital sound is obtained the wow and flutter reduction can be applied. Here the known non-uniform resampling techniques can be used [20].

4. Experiments

The experiments aim at the comparison of the new method for wow and flutter determination and reduction with the known approaches. Two factors are taken into consideration. The first one is the maximal modulation frequency (fmax) which can be captured in the determined PVC. The second one is the precision of the determined PVC.

The preliminary study of the fmax limitation for the new method was done by the authors of this paper [8]. It was shown that the limitation depends on the film format and the movie tape velocity, which can be expressed using either frame per second (fps) or perforation per second (pps) characteristics. The determined values of the fmax are given in table 1 and 2.

Table 1 Frame-based fmax limitations of the algorithm for the local tape shrinkage analysis [8]

Table 2 Perforation-based fmax limitations of the algorithm for the local tape shrinkage analysis [8]

As the 35 mm film is one of the most popular formats in film archives [3] it was chosen for the experiments.  Additionally, as the perforation-based fmax is higher for this film format, the perforation-based processing was used in the performed simulations.

Among the reported audio-based algorithms for the PVC determination (see Section 2) only tree have been studied in terms of the fmax limitation [19]. Those are: the power line hum tracker, the centre of gravity (COG) tracker and the high frequency bias tracker. The results of this study are given in table 3. In the table the Fs stands for the sampling frequency of the digitized audio recordings, whereas M represents the processing frame length.

Table 3. fmax limitations for the wow and flutter tracking algorithms using audio analysis [19]

Among those tree algorithms the power line hum tracker is the most appropriate for the comparison with the new method. It is because it can be used almost automatically, avoiding problems with the wow versus vibrato comparison. Additionally, power line hum can be easily found in many of the film soundtracks (which is true for the recordings from the Polish film archives).

The experiments were performed as follows. First the wow and flutter simulations were prepared. For the power line hum tracker 4 sec long sound files with 50 Hz and 60 Hz tones were prepared, simulating the European- and USA-originated recordings respectively. For the new method 100 movie frames (digital images) in the 35 mm format were prepared, which corresponds to a 4 sec long audio and video recording, assuming the 25 fps tape velocity. As the typical tape resolution lies between 60 to 80 lines per mm [5], which represents the range from 1524 DPI to 2032 DPI, the 3600 DPI image resolution was chosen to satisfy the most demanding cases.

Both the sound files and the movie frames were frequency modulated using a complex characteristic with the limited maximal frequency. The characteristic was generated as the white Gauss noise, low-pass filtered with the cut-off frequency equalling the fmax limitation. Five fmax values were studied: 0.5 Hz, 6 Hz, 12 Hz, 19 Hz and 38 Hz. First two correspond to the wow definition (fm form 0.5 Hz to 6 Hz). Third one is the fmax limitation of the power line hum tracker (see table 3). The fourth is the fmax limitation of the new method (see table 2) whereas the last value doubles that limitation. Frequency modulation was performed using the non-uniform resampling: one-dimensional for the sound files and two-dimensional for the movie frames. Two values of the maximal, allowable, relative deviation of the modulation characteristics (i.e. modulation depth) were used in the experiment: 1.1 and 1.01. The first value was used for simulating gross pitch variations whereas the second value for simulating  smaller ones. The simulations were repeater 100 times for each algorithm, for all fmax values, and for both maximum deviation values. The mean square error (MSE) between the simulated and the determined PVCs was calculated for each simulation. Then the results were summarized using the mean and standard deviation values for all of the calculated MSEs.

Results obtained using the power line hum tracker are given in tables from 4 to 7. First the results for the simulations involving the 50 Hz tone are given. They are followed by the results for the sound files with the 60 Hz tone. In each case the results from the simulations with the gross deviations are presented first.

Table 4. MSE of the PVCs determined using the power line hum tracker, 50 Hz tone, maximal allowable deviation 1.1

Table 5. MSE of the PVCs determined using the power line hum tracker, 50 Hz tone, maximal allowable deviation 1.01

Table 6. MSE of the PVCs determined using the power line hum tracker, 60 Hz tone, maximal allowable deviation 1.1

Table 7. MSE of the PVCs determined using the power line hum tracker, 60 Hz tone, maximal allowable deviation 1.01

The results obtained for the new method are given in tables 8 and 9.

Table 8. MSE of the PVCs determined using the local tape shrinkage analysis, maximal allowable deviation 1.1

Table 9. MSE of the PVCs determined using the local tape shrinkage analysis, maximal allowable deviation 1.01

To visualise the obtained results better additional charts are given. Fig. 8 and 9 present the box-plots of the mean MSEs obtained for the high and low deviations respectively.

Fig. 8 Mean MSEs obtained for the simulations with the high relative deviations

Fig. 9 Mean MSEs obtained for the simulations with the small deviation.

For the simulations involving samples with the higher relative deviations it can be seen that the new method provides better results. The overall MSE, calculated for all of the examined fmax values, is smaller for this method comparing to the total MSE of the power line hum tracker. For both algorithms the MSE increases with the higher fmax values, however, the degree of increase is smaller for the new method.

For the simulations involving samples with the smaller relative deviations the results are different. The overall MSE is higher for the new method. It can be seen that the mean MSEs are the same for all of the examined fmax values (here only the standard deviation values allows to differentiate results). The high and constant values of the mean MSEs are caused by the precision limitation of the proposed method. Using the 3600 DPI resolution images the distance between successive perforation holes equals 670 pixels. Knowing that the maximal allowable deviation is only 1.01, the maximal length in the modulated perforations’ distances equals 7 pixels, and in most cases is less than that. Comparing this with the limited precision of the perforation distance evaluation of the new method, which is +/- 2 pixels, explains the reported constant value of the MSE.

5. Conclusions

The new method presented in this paper allows to determine course of wow in the entire range of fm. The flutter course can be determined up to the to the fmax equalling around 19 Hz. Additionally, when this limitation is exceeded the tracking error increases slowly comparing to errors obtained for the power line hum tracker.

For the gross pitch variations the new method allows for a better accuracy then the power line hum tracker. However, for the small pitch variations precision of the new method is lower. Low precision is caused by the new method accuracy limitation due to the movie tape images resolution.

The new method can be used automatically providing the movie tape scans are prepared properly. Additionally the new method allows to overcome the problems with the comparison between natural and parasitic pitch variations (i.e. wow versus vibrato).

The new method can be used to restore obsolete and rare archival recordings.

6. Summary

???

Acknowledgements

This work was supported by the Polish Government research resources for years 2007-2009 as the research project no. N N517 4381 33 (decision no. 4381/B/T02/2007/33).

References

[1] AES Std. 6-1982, Method for Measurement of Weighted Peak Flutter of Sound Recording and Reproducing Equipment, Audio Engineering Society, New York, 2003.

[2] E. D. Daniel, C. D. Mee, M. H. Clark, Magnetic Recording: The First 100 Years, IEEE Press, New York, 1999.

[3] P. Read and M. P. Meyer, Restoration of Motion Picture Film, Butterworth Heinemann, Oxford, UK, 2000.

[4] E. S. Busby, Analog Tape Recording, in: J. Whitaker, B. K. Benson (Eds.), Standard Handbook of Audio and Radio Engineering, McGraw-Hill, New York, 2003, pp. 6–21-6–42.

[5] I. Malecki, Technika Nagrywania i Odtwarzania Dzwiekow, PWT, Warszawa 1953. (in Polish)

[6] A. Czyzewski, A. Ciarkowski, A. Kaczmarek, J. Kotus, M. Kulesza, P. Maziewski, DSP Techniques for Determining “Wow” Distortion, Journal of the Audio Engineering Society 55 (4) (2007) 266-284.

[7] P. Pastuszak, MTS Pilote Tone Tracking for “Wow” Determination, Archives of Acoustics 33 (1) (2008) 117-123.

[8] P. Maziewski, A. Kupryjanow, A. Czyzewski, Drift, Wow and Flutter Measurement and Reduction in Shrunken Movie Soundtracks, presented at the 124th AES Convention, Amsterdam, The Netherlands, 17-20 May 2008, preprint no. 7392.

[9] M. Gerzon, Don’t Destroy the Archives!, unpublished (1992); available at ww.waves.com/german/htmls/service/faq/dont.html (accessed 2006 Oct.).

[10] S. J. Godsill, The Restoration of Degraded Audio Signals, Ph.D. dissertation, Cambridge University, Cambridge, UK, 1993.

[11] S. J. Godsill, P. Rayner, The Restoration of Pitch Variation Defects in Gramophone Recordings, Proc. IEEE Workshop on Applications of Signal Processing to Audio and Acoustics, Mohonk, NY, 17-20 October 1993, pp. 148-151.

[12] S. J. Godsill, Recursive Restoration of Pitch Variation Defects in Musical Recordings, Proc. IEEE Int. Conf. on Acoustics, Speech and Signal Processing 2, Adelaide, Australia, 19-22 April 1994, pp. 233-236.

[13] S. J. Godsill, P. Rayner, Digital Audio Restoration - A Statistical Model-Based Approach, Springer, London, 1998, Chapter 8, pp. 171–190.

[14] S. J. Godsill, G. Reid, and C. Hicks, “Restoration of Smooth Pitch Variations over Long Timescales,”  presented at the Joint Technical Symposium, Toronto, Canada, 24–26 June 2004; available at http://www.jts2004.org/english/proceedings/Reid.htm (accessed October 2008).

[15] J. Nichols, An Interactive Pitch Defect Correction System for Archival Audio, Proc. AES 20th Int. Conf., Budapest, Hungary, 5-7 October 2001, paper no. 1941.

[16] J. Nichols, High Performance, Low-Cost Wax Cylinder Transcription System, Proc. AES 20th Int. Conf., Budapest, Hungary, 5-7 October 2001, paper no. 1937.

[17] J. Howarth, P. Wolfe, Correction of Wow and Flutter Effects in Analog Tape Transfers, presented at the 117th AES Convention, San Francisco, USA, 28-31 October 2004, preprint no.6213.

[18] P. Wolfe, J. Howarth, Nonuniform Sampling Theory in Audio Signal Processing, presented at the 116th AES Convention, Berlin, Germany, 8-11 May 2004, preprint no. 6123.

[19] P. Maziewski, Modulation Frequency Constrains on Wow and Flutter Determination, Archives of Acoustics 33 (1) (2008) 125-131.

[20] P. Maziewski, Wow defect reduction based on interpolation techniques, Bulletin of the Polish Academy of Science: Technical Sciences 54 (4) (2006) 469-477; available at http://bulletin.pan.pl/(54-4)469.pdf (accessed October 2008).

[21] V. Fadeyev, C. Haber, A novel application of high energy physics technology to the problem of audio preservation, Nuclear Instruments and Methods in Physics Research A 518 (2004) 456-462.

[22] E. Brun, A. Hassaine, B. Besserer, E. Decenciere1, Restoration of Variable Area Soundtracks, IEEE International Conference on Image Processing, ICIP 2007, San Antonio, Texas, USA, 16-19 September 2007.

[23] D. Richter, D. Poetsch, A, Kuiper, Localization of Faults in Multiple Double Sided Variable Area Code Sound Tracks on Motion Picture Films Using Digital Image Processing, 13th International Czech-Slovak Scientific Conference Radioelektronika, 2003.

Vitae

Andrzej Czyzewski was born in Gdansk, Poland, in 1956. He received an M.Sc. degree in sound engineering from the Gdansk University of Technology in 1982, a Ph.D. degree in 1987, and a D.Sc. degree in 1992. He was granted the title of professor in 1999 and he has held the position of full professor at Gdansk University since 2002. His main interest is in research and education in the domain of multimedia and studio recording as well as in modern information and communication technologies.

Przemyslaw Maziewski was born in Suwalki, Poland, in 1978. He received an M.Sc. degree from the Faculty of Electronics, Telecommunications and Informatics of the Gdansk University of Technology, Poland. His thesis was related to the problem of sound virtualization techniques implemented in stereo headphones. His fields of interests are related to audio recording, digital signal processing, and sound archiving. Currently he continues his studies at the Gdansk University of Technology as a Ph.D. candidate.

Adam Kupryjanow was born in Bialystok, Poland in 1983. He received his M.Sc. degree in 2007 from the Faculty of Electronics, Telecommunications and Informatics of the Gdansk University of Technology, Poland. His thesis was related to optical soundtrack reconstruction. Currently he is a student of Ph.D. studies at the Gdansk University of Technology

Figures

Fig. 1 Example of a badly produced movie tape splice

Fig. 2 Tape shrinkage effects on the movie tape transportation system [5]

Pdist Pdist2 2 Pdist3 Pdist4 Pdist5 Pdist6 t1 t2 t3 t4 t5 t6

Fig. 3 Example of a typical 35 mm movie frame

Fig. 4 Algorithm for calculating the distances between successive perforation holes in one movie frame

Fig. 5 Example of determined distances between successive perforation holes for the 35 mm film (one frame)

Fig. 6 Example of determined distances between successive movie frames for the 35 mm film

Fig. 7 Scheme of the algorithm for translating the image of the optical soundtrack to the digital sound

Fig. 8 Mean MSEs obtained for the simulations with the high relative deviations

Fig. 9 Mean MSEs obtained for the simulations with the small relative deviation

Tables

Table 1 Frame-based fmax limitations of the algorithm for the local tape shrinkage analysis [8]

| Film format [mm] | 8 | 16 | 35 | 65/70 |
| --- | --- | --- | --- | --- |
| Tape velocity [fps] | 16 | 24 | 24 | 24 |
| fmax [Hz] | <3.2 | <4.8 | <4.8 | <4.8 |

Table 2 Perforation-based fmax limitations of the algorithm for the local tape shrinkage analysis [8]

| Film format [mm] | 8 | 16 | 35 | 65/70 |
| --- | --- | --- | --- | --- |
| Tape velocity [pps] | 16 | 24 | 96 | 120 |
| fmax [Hz] | <3.2 | <4.8 | <19.2 | <72 |

Table 3. fmax limitations for the wow and flutter tracking algorithms using audio analysis [19]

| Algorithms and their settings | fmax [Hz] |
| --- | --- |
| Power line hum tracker; M = 8 Sa; Fs =200 Hz; | < 12.5 |
| COG tracker; M = 1024 Sa; Fs =44100 Hz; | < 21.5 |
| High-frequency bias tracker; M = 1024 Sa; Fs =192000 Hz | < 93.75 |

Table 4. MSE of the PVCs determined using the power line hum tracker, 50 Hz tone, maximal allowable deviation 1.1

| fmax [Hz] | 0.5 | 6 | 12 | 19 | 38 |
| --- | --- | --- | --- | --- | --- |
| Mean Value (Std. Deviation) | 1,48E-05 (1,10E-05) | 8,38E-05 (2,78E-05) | 2,04E-04 (4,16E-05) | 3,63E-04 (6,99E-05) | 5,03E-04 (9,40E-05) |

Table 5. MSE of the PVCs determined using the power line hum tracker, 50 Hz tone, maximal allowable deviation 1.01

| fmax [Hz] | 0.5 | 6 | 12 | 19 | 38 |
| --- | --- | --- | --- | --- | --- |
| Mean Value (Std. Deviation) | 1,29E-05 (8,78E-06) | 1,36E-05 (8,05E-06) | 1,69E-05 (7,06E-06) | 1,96E-05 (7,10E-06) | 1,91E-05 (6,70E-06) |

Table 6. MSE of the PVCs determined using the power line hum tracker, 60 Hz tone, maximal allowable deviation 1.1

| fmax [Hz] | 0.5 | 6 | 12 | 19 | 38 |
| --- | --- | --- | --- | --- | --- |
| Mean Value (Std. Deviation) | 1,69E-05 (1,19E-05) | 1,31E-04 (7,18E-05) | 2,61E-04 (5,14E-05) | 4,00E-04 (7,27E-05) | 5,27E-04 (8,20E-05) |

Table 7. MSE of the PVCs determined using the power line hum tracker, 60 Hz tone, maximal allowable deviation 1.01

| fmax [Hz] | 0.5 | 6 | 12 | 19 | 38 |
| --- | --- | --- | --- | --- | --- |
| Mean Value (Std. Deviation) | 8,27E-06 (6,23E-06) | 8,69E-06 (5,64E-06) | 1,18E-05 (5,91E-06) | 1,24E-05 (5,49E-06) | 1,38E-05 (5,11E-06) |

Table 8. MSE of the PVCs determined using the local tape shrinkage analysis, maximal allowable deviation 1.1

| fmax [Hz] | 0.5 | 6 | 12 | 19 | 38 |
| --- | --- | --- | --- | --- | --- |
| Mean Value (Std. Deviation) | 5,09E-05 (4,74E-05) | 5,02E-05 (5,43E-05) | 8,09E-05 (0,000154) | 9,05E-05 (0,000153) | 1,01E-04 (0,000179) |

Table 9. MSE of the PVCs determined using the local tape shrinkage analysis, maximal allowable deviation 1.01

| fmax [Hz] | 0.5 | 6 | 12 | 19 | 38 |
| --- | --- | --- | --- | --- | --- |
| Mean Value (Std. Deviation) | 2,32E-05 (2,99E-07) | 2,32E-05 (2,63E-07) | 2,32E-05 (2,84E-07) | 2,32E-05 (2,22E-07) | 2,32E-05 (2,92E-07) |